Your Company Name

OPERATIONAL MANUAL

| Issued Date |  | Issued By |  |
| --- | --- | --- | --- |
| Date Checked |  | Checked by |  |
| Released Date |  | Released by |  |
| Approval Date |  | Approved by |  |

Document Reference: Operations Manual for Operational Authorisation

Issue: Ver 1.0 – 1 January 2022

Document Author: your name

Accountable Manager:  company owner/director

Contents

List of Tables	5

List of Images	5

1.Introduction	7

1.1 Definitions	7

1.1.1 Definition of Terms	7

1.1.2 Abbreviations and Acronyms	8

1.2 System for Amendment and Revision of the OM	9

1.2.1 Amendment Table	9

1.2.2 List of Refence Documents	10

1.3 Record of Revisions with effective dates.	10

1.4 List of effective pages	10

1.5 Purpose and Scope of Operations Manual	10

1.6 Safety Statement	10

1.6.1 Safety Assurance	10

1.6.2 Safety Training	11

1.7 Approval of Signature	11

2.Description of the UAS Operator’s Organisation	11

2.1 Organisation overview - Organigram	11

2.2 Aeronautical Decision Making	12

2.3 Procedures & Checklists	12

2.4 Recurrent Training	13

2.5 Training of Staff involved in Operation.	13

2.6 Maintenance	13

2.7 Crew	13

3.Concept of Operations:	14

3.1 Nature of the operation and associated risks	14

3.1.1 Topographical Survey conducted with Multi-Rotor.	14

3.1.2 Bridge/Roof/Façade Inspections conducted with Multi-Rotor Drones.	14

3.1.3 Coastal Erosion/Defence/Flooding/Subsidence conducted with Multi-Rotor Drones	14

3.2 Operational environment and geographical area for the intended operations	14

3.3 Type of Operations	15

3.3.1 Duration of the Operations	16

3.3.2 Area of Operations	16

3.3.3 Typical Flight Missions uncontrolled Airspace	23

3.3.4 Level of Involvement (LoI)	29

3.3.5 Normal Operation Strategy	30

3.3.6 Standard Operating Procedures	32

3.4 SORA - Specific Operating Risk Assessment for Risk analysis and methods for reduction of identified risks	36

3.4.1 Applicability	37

3.4.1.2 Company RAMS (Risk assessments and Method Statements).	37

3.4.2 Ground Risk	37

3.4.3 Air Risk	42

3.4.4 SAIL	48

3.4.5 OSOs	49

3.5 Technical means used.	71

3.5.1 Multi-Rotor DJI Mavic 2 Enterprise Advanced	72

3.6 Competency, Duties and Responsibilities	73

3.6.1 Head of Drone Operations.	73

3.6.2 Safety Manager	74

3.6.3 Head of Training	74

3.6.4 Maintenance Manger	75

3.6.5 Remote Pilot/Chief	75

3.6.6 Remote Pilot	77

3.6.7 Visual Observer (VO - Optional)	78

3.6.8 Competency - Fitness for Duty	78

3.6.9 Competency - Shift & Working Hours	78

3.6.10 Competency - Medication & Wellbeing.	79

3.7 Training	79

3.8 Risk analysis and methods for reduction of identified risks	80

3.9 Maintenance	80

4. Normal procedures.	81

4.1 General procedures valid for all operations:	81

4.2 Procedures peculiar to a single operation	82

5. Contingency procedures	83

5.1 General procedures valid for all operations	83

5.1.1 Sudden Change in Weather into adverse conditions Actions.	83

5.1.2 UAS Departs the Flight Geography Actions	84

5.1.3 Sudden Mission Termination because of Compass Failure Actions.	84

5.1.4 Telemetry Loss during Flight Mission Actions	85

5.1.5 Interference Avoidance with Manned Aircraft Actions.	85

6. Emergency procedures	86

6.1 List of Possible Emergency’s	86

6.2 Fail Safe Procedure	86

6.3 Force Landing Procedure	87

6.4 Software Malfunction	88

6.5 Transmitter Failure/Frequency Interference	89

6.6 Loss of Propulsion/Motor or Propeller Failure	90

6.7 Aircraft Battery Failure	91

6.8 Loss of GPS Signal	92

6.9 Low Magnetic Field	93

6.10 Public Encroachment	94

6.11 Aircraft Incursion	95

6.12 Fly Away	96

6.13 Pilot Incapacitation	97

6.14 Fire Ground Equipment	98

6.15 Fire In Flight	99

7. Emergency response plan (ERP)	100

7.1 UAS Loss of Containment Emergency Actions	101

7.2 Immediately Post-Incident Actions	101

7.3 Environmental Emergency Actions	101

7.4 UAS Loss of Containment Emergency Actions.	102

8. Security	102

8.1 Operational	102

8.2 Data Protection	103

9. Guidelines to minimise nuisance and environmental impact referred to in UAS.SPEC.050(a)(v);	104

10. Occurrence reporting procedures according to Regulation (EU) No 376/2014.	105

10.1 Reporting	105

10.2 Starting a report.	106

10.3 Reporting Procedure.	106

11. Record-keeping procedures	108

11.1 Pilot Logbook	108

11.2 Flight Logbook	109

11.3 Battery Logbook	110

11.4 Drone Maintenance Logbook	111

# List of Tables

Table 1: Abbreviation	9

Table 2: Amendment Record	9

Table 3:List of References	10

Table 4: Record of Revisions with effective dates	10

Table 5: Company Structure & Personnel	12

Table 6: Flight Mission parameters	23

Table 7: Controlled airspace parameters	23

Table 8: Emergency Phone Contacts	36

Table 9: Ground Risk Mitigation Robustness	39

Table 10: Summary of the OSO Analysis	52

Table 11: List of Pilots	73

Table 12: Pilot Logbook	108

Table 13:: Flight Logbook	109

Table 14:Battery Logbook	110

Table 15: Maintenance Logbook	111

# List of Images

Image 1: Shannon FIR (Flight Information Region)	16

Image 2: Population Assemblies	17

Image 3: Ground Infrastructure by County	18

Image 4: Example of Rural Aerodrome	19

Image 5: LTE Connectivity Coverage for Vodafone	20

Image 6: LTE Connectivity Coverage for Three	20

Image 7: Temperature Average	21

Image 8: Daily Chance of Precipitation	21

Image 9: Average Wind Speed in Ireland	22

Image 10: Humidity Comfort Levels	22

Image 11: Hours of Daylight	22

Image 12:Typical Flight Mission 2D	24

Image 13:Typical Flight Mission 3D	24

Image 14:Typical Linear Mission	25

Image 15:Typical Waypoint Mission - Uncontrolled Airspace	25

Image 16: Typical 2D Mission - Controlled Airspace	26

Image 17:Typical Linear Mission - Controlled Airspace	26

Image 19:Typical 3D Mission - Controlled Airspace	27

Image 18:Typical Waypoint Mission - Controlled Airspace	27

Image 20: Containment/Geo-Fence/Boundary Box in DJI App	28

Image 21:Geo-Fence Parameters Diagram	28

Image 22:DJI Flysafe Geo-Zones	29

Image 23:Loss of Connection with DJI App Software	34

Image 24: Determination of Robustness Level SORA EU 2019/947 (Table 4)	36

Image 27:Mavic 2 Enterprise Advanced	72

Image 28:Ground Risk / Air Risk Model	80

Image 29:Operational Volume	83

# 1.Introduction

Explain what this Manual is about and a very brief explanation of the type of Drone Operations and a brief outline of your company.

## 1.1 Definitions

The relevant definitions, terms and abbreviations used in this manual are described below:

### 1.1.1 Definition of Terms

Accident – An unplanned event or series of events that results in death, injury, or damage to, or loss of, equipment or property.

Cause – One or several mechanisms that trigger the hazard that may result in an accident or incident: the origin of a hazard.

Control – A current, planned, or proposed means to reduce or eliminate a hazard’s causes or effects.

Cruise – Defined as that phase of the flight that happens BVLOS, different than:

• Take-off & departure    • Approaches & landing

Effect – The real or credible harmful outcome that has occurred or can be expected if the hazard occurs in a defined system state.

Hazard – Any real or potential condition that can cause injury, illness, or death to people; damage to or loss of a system, equipment, or property; or damage to the environment. A hazard is a prerequisite to an accident or incident.

Incident – An occurrence other than an accident that affects or could affect the safety of operations.

Likelihood – The estimated probability or frequency, in quantitative or qualitative terms, of a hazard’s effect or outcome.

Mitigation Measure (or Safety Requirement) – Action required to reduce the associated risk by lessening the severity of the resulting mishap or lowering the likelihood that a mishap will occur.

Risk (or Safety Risk) – The combination or composite of the predicted severity and likelihood of occurrence of a hazard’s effect or outcome. The expected value of loss resulting from the hazard.

Severity – The consequence or impact of a hazard’s effect or outcome in terms of degree of loss or harm to include death, injury, damage to or loss of equipment or property, damage to the environment, or monetary loss.

System State – An expression of the various conditions, characterized by quantities or qualities, in which a system can exist.

### 1.1.2 Abbreviations and Acronyms

| Abbreviation / Acronym | Meaning |
| --- | --- |
| A/C | Aircraft |
| AGL | Above Ground Level |
| AED | Automatic External Defibrillator |
| AM | Accountable Manager |
| AMSL | Above Mean Sea Level |
| BVLOS | Beyond Visual Line of Sight |
| C2 | Command and Control Link |
| C3 | C2 + any communication required |
| CoG | Centre of Gravity |
| CONOPs | Concept of Operations |
| CPR | Cardiopulmonary Resuscitation |
| DAA | Detect and Avoid |
| DUTO | Declared Unmanned Aerial System training Organization. |
| EASA | European Aviation Safety Agency |
| EMC | Electromagnetic Compatibility |
| ESC | Electronic Speed Controller |
| ERP | Emergency Response Plan |
| FOM | Flight Operations Manager |
| GNC | Guidance Navigation and Control |
| GPS | Global Positioning System |
| HL | Heavy Lift |
| HOD | Hook On Device |
| IMU | Inertial measurement unit |
| LCD | Liquid Crystal Display |
| LED | Light Emitting Diode |
| L/H | Left-Hand |
| LTE | Long Term Evolution |
| MM | Maintenance Manager |
| MT | Maintenance Technician |
| NAA | National Aviation Authority |
| N/A | Not Applicable |
| N/C | Not Compliant |
| OM | Operations Manual |
| OSO | Operational Safety Objective |
| PIC | Pilot in Command |
| PPE | Personal Protective Equipment |
| R/C | Remote Control |
| RED | Radio Equipment Directive |
| RF | Radio Frequency |
| RFD | Radio Frequency Distribution |
| R/H | Right-Hand |
| RP | Remote Pilot |
| RTL | Return to Launch |
| SAIL | Specific Assurance and Integrity Level |
| SM | Safety Manager |
| SP | Safety Pilot |
| SRD | Short Range Device |
| TEM | Threat and Error Management |
| TMPR | Tactical Mitigation Performance Requirement |
| TRA | Temporary Restricted Area |
| UAS | Unmanned Aircraft System |
| UAV | Unmanned Aerial Vehicle |
| USB | Universal Serial Bus |
| VLOS | Visual Line of Sight |
| VMC | Visual Meteorological Conditions |
| VO | Visual Observer |
| VTOL | Vertical Take-off and Landing |

Table 1: Abbreviation

## 1.2 System for Amendment and Revision of the OM

This Operations Manual is a living document for the guidance of all personnel involved in flight operations and as such is to be continuously reviewed and updated as necessary due to:

- Outcome of the study of incidents / accident reports

- Constructive submissions on the content of this manual as operational and   technical content changes. All suggested amendments to this manual are to be submitted in writing to Your Company Name Flight Operations Manager completed with supportive evidence to help achieve the maximum safety and efficiency.

- Changes in regulations

### 1.2.1 Amendment Table

| Amendment/Revision/Issue Number | Date | Amended By | Signed |
| --- | --- | --- | --- |
| Operations Manual: Version Number | Description of Change |  |  |

Table 2: Amendment Record

### 1.2.2 List of Refence Documents

| Number | Name of Document | Reference ID | Year | Revision |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |

Table 3:List of References

## 1.3 Record of Revisions with effective dates.

| Revision/Issue Number | Date | Amended By | Signed |
| --- | --- | --- | --- |

Table 4: Record of Revisions with effective dates

## 1.4 List of effective pages

(List the Effective pages unless the entire manual is re-issued, and the manual has an effective date on it)

## 1.5 Purpose and Scope of Operations Manual

Write the main purpose of this document and give a brief description of the different parts of this document.

“The Your Company Name Operations Manual describes the organisation of our company, the concept of operations, the Normal Procedures, Contingency Procedures, Emergency Procedures, Emergency Response Plan, Security of the modification or interference of aircraft systems, personnel, flight operations and procedures by which the company conducts its business Unmanned Air System operations. The contents of this document do not override the requirement for continual review to ensure any new or amended regulation published from time to time by the relevant National Aviation Authorities addressed by this document.”

## 1.6 Safety Statement

To include a Statement to say that this OM complies with the relevant requirements of Regulation (EU) 2019/947 and with the operational authorisation received to Your Company Namefrom the Irish Aviation Authority (IAA).

“I hereby acknowledge that I have received the instructions, procedures and data contained in the OM complies with the relevant requirements of Regulation (EU) 2019/947 and with the authorisation or the terms of approval of the light UAS operator certificate (LUC), in the case of a LUC holder, and contains instructions that are to be complied with by the personnel involved in flight operations).”

### 1.6.1 Safety Assurance

In order to operate to the highest standards of safety, aircraft will be physically checked for defects before every flight and routine maintenance will take place on each aircraft after each flight or after the number of hours recommended by the aircraft manufacturer.  Aircrew procedures from the start of operations to the end of operations will be followed implicitly and any incident will be recorded in the appropriate NAA documentation.

### 1.6.2 Safety Training

Safety training will take place every quarter within Your Company Name . It will involve reviewing procedures as well as looking at flight safety incidents that have occurred both internal and external to the company’s sphere of operations. Changes in the safety requirement will be recorded and disseminated to each staff member who will be prohibited from flying until the ‘changes document’ has been read and signed for.

## 1.7 Approval of Signature

The Your Company Name Operations Manual approval signature is to confirm that the contents of this document do not override the requirement for continual review to ensure any new or amended regulation published from time to time by the relevant National Aviation Authorities addressed by this document. The approval signature is also to confirm the Safety Statement above.

| Signed |  |
| --- | --- |
| Date | January 2022 |
| Accountable Manager |  |
| Your Company Name |  |

For and on behalf of Your Company Name a company registered Company No:

Company address:

# 2.Description of the UAS Operator’s Organisation

Write a Brief Description of your company outlining your company strengths.

(a) The organisational structure and designated individuals. Description of the operator’s organisational structure, including an organisational chart showing the different departments, if any (e.g. flight/ground operations, operational safety, maintenance, training, etc.) and the head of each department;

(b) Duties and responsibilities of the management personnel,

(c) Duties and responsibilities of remote pilots and other members of the organisation involved in the operations (e.g. payload operator, ground assistant, maintenance technician, etc.).

## 2.1 Organisation overview - Organigram

Expand this if you need to.

| Name of person | Role of Person | Contact – email & Phone |  | Remarks |
| --- | --- | --- | --- | --- |

Table 5: Company Structure & Personnel

## 2.2 Aeronautical Decision Making

Your Company Name UAS Remote Pilots are taught about the characteristics of analytical and naturalistic aeronautical decision making in the different phases of the operation.

In the internal training the pilots are provided with information and techniques to improve their aeronautical decision-making capabilities and make right decisions during the flight-preparations, abnormal and emergency situations.

## 2.3 Procedures & Checklists

Procedures and Checklist have been developed and improved through more than 2000hours of VLOS experience. Their improvement is continuous, and it is performed based on the study of the performance of the team and of occurrences related to the operation.

Withing the standard operational procedures the working and resting hours of the pilots are defined, as well as an evaluation of the pilot fitness using the acronym IMSAFE.

- Illness

- Medication

- Stress

- Alcohol

- Fatigue

- Emotions

## 2.4 Recurrent Training

After the investigation of occurrences new procedures may be developed. The UAS Remote Pilots are informed and trained on the new procedures and are encouraged to exercise this and the current ones safely during the operations. Your Company Name UAS Remote Pilots will conduct a training exercise once every 6 months at a minimum. During the exercises all ERP scenarios will be exhausted.

## 2.5 Training of Staff involved in Operation.

Your Company Name are not a Recognized Entity with the Irish Aviation Authority. All Drone Training and Licencing must be completed with a third-party company. This external company must be a Declared Unmanned Aerial System training Organization. (DUTO). Their experience and knowledge base must be in the field of VLOS/BVLOS Operations. This external company will conduct Training for VLOS operations covering STS-01 Operations and BVLOS operations Covering STS-02 operations. They will also have the ability to Conduct Flight Exams to certify UAS Remote Pilots. If a lesser Certificate is required, this external company will have the ability and knowledge to train and certify all Your Company Name Pilots.

## 2.6 Maintenance

The General Maintenance philosophy of Your Company Name is broken down into three factors.

- General maintenance

- Preventive maintenance

- Predictive maintenance

See Table 2 for the Your Company Name Maintenance Manual with reference to the Maintenance of the Aircraft.

## 2.7 Crew

As Stated in OM for reference of current document version see Table 2.

# 3.Concept of Operations:

## 3.1 Nature of the operation and associated risks

Below is a list of Drone Operations that we conduct within Your Company Name

### 3.1.1 Topographical Survey conducted with Multi-Rotor.

We conduct Topographical Surveys with the use of Multi-Rotor Drones for the purpose of capturing photos which enables Your Company Name  to make 2D drawings for Clients. Quarries, Road Networks, Rail Networks, Waterways, The general hazards for Air and Ground Risk in which we encounter could be Birds, Poles, Live Trains passing at high speed, Live Cars Moving at high speed, wake turbulence is a possibility, vegetation, or livestock, moving water and walkers. For further risk and mitigation please see Section 3.4 SORA of this manual.

### 3.1.2 Bridge/Roof/Façade Inspections conducted with Multi-Rotor Drones.

We conduct Bridge/Roof/Façade inspections with the use of a Multi-Rotor Drone for the purpose of capturing photos to identify any possible defects that need to be address by the bridge/Roof/Façade owners. The general hazards for Air and Ground Risk in which we encounter could be Wi-Fi & Microwave antennas, Birds, Poles, Live Trains passing at high speed, Live Cars Moving at high speed, wake turbulence is a possibility, vegetation, or livestock, moving water and walkers. For further risk and mitigation please see Section 3.4 SORA of this manual.

### 3.1.3 Coastal Erosion/Defence/Flooding/Subsidence conducted with Multi-Rotor Drones

We conduct Coastal inspections with the use of a Multi-Rotor Drone for the purpose of capturing photos to identify any possible Erosion, Coastal Defence Issues or defects in the Defence products, any possible Subsidence of Sand Banks that may be a hazard to Buildings, People or Animals. Areas close to the coast like rail networks that could be potentially under threat of flooding. The general hazards for Air and Ground Risk in which we encounter could be Birds, Poles, Live Trains passing at high speed, Live Cars Moving at high speed, wake turbulence is a possibility, vegetation, or livestock, moving water and walkers. For further risk and mitigation please see Section 3.4 SORA of this manual.

## 3.2 Operational environment and geographical area for the intended operations

The Operational environment for drone operations in section 3 of this Manual, the Overflown area will be similar, Urban and Rural. Populated and sparsely populated areas in Urban and Rural areas.

Urban

The Topography of the area for Urban will be working within and around Cities, Towns, Villages, Harbours and Ports. Within Cities and Towns there would be built-up areas of buildings roads, rail, waterways, and park areas. The Ground Risk buffer will utilise the 1:1 rule when working in and around uninvolved persons. It is reasonable to consider that most of the non-involved persons will be located within a building.

Within the Urban Areas there could be a possibility of some type of electromagnetic environment, were Wi-Fi antennas and Microwave antenna may emit an electromagnetic signal that may interfere with the Drone Communications. The Emergency Immediate action for this is to fly out of the general area either in an upward motion or a backward motion. If this fails, the Drone has a safety feature when it loses communications with the remote controller it is designed by default to go into Return to Home function.

Rural

The Topography for Rural and sparsely populated areas Villages, small rows of houses along a country road. The Ground Risk buffer will utilise the 1:1 rule when working in and around uninvolved persons. The Air risk can increase for the possibility of unregistered Aerodromes. Remote Pilots will complete a Google Earth Pre-deployment reconnaissance on the Area to be flown for any unregistered Aerodromes within a 5km radius of Drone operational area. Most Rural Areas will be Uncontrolled Airspace this by nature reduces the Air risk from aircraft. But still does poses a different air risk as now all private aircraft will be conducting Visual Flight rules (VFR) flights. Depending on the type of Drone Operation an Aircraft Observer (AO) may be required.

Within Rural Areas you could be operating close to or around Wi-Fi and Microwave Antenna Sites. Some of these sites could emit an electromagnetic energy that could break communications between the Drone and the Remote Controller. The Emergency Immediate action for this is to fly out of the general area either in an upward motion or a backward motion. If this fails, the Drone has a safety feature when it loses communications with the remote controller it is designed by default to go into Return to Home function. When flying close to or within a 100meter radius of an electrical substation, the Pilot will at no time fly closer than 60meters to this obstacle. When flying close to or around an area that has high tension power cables the Pilot will not fly directly over these cables or with 30meters of them. These Cables can perform what is called an electrical Arc. If a drone is stuck by this arc it will result in the loss of operations.

## 3.3 Type of Operations

Your Company Name intends to carry out VLOS UAS operations.

- (A) Topographical Survey conducted with Multi-Rotor Drones.

- (B) Bridge/Roof/Façade Inspections conducted with Multi-Rotor Drones.

- (C) Coastal Erosion/Defence/Flooding/Subsidence conducted with Multi-Rotor Drones.

- (D) Electrical Substations/Phone Mast/Electrical Power cables with Multi-Rotor Drones.

Payload: RBG Camera

### 3.3.1 Duration of the Operations

The Duration of Operations will be as long as the Operational Authorization is valid for. In the case of modifications, the IAA will be contacted.

### 3.3.2 Area of Operations

The area of operations is defined by Shannon FIR, with a maximum altitude of 120m (400ft) AGL, see Figure 2.

Image 1: Shannon FIR (Flight Information Region)

#### 3.3.2.1 Ground Class

The volume of operations is defined above a sparsely populated environment & densely populated environment. Flight Planning and Flight mission will be planned in such a manner to avoid assemblies of houses or populated areas as much as possible. When flight planning it is more than likely there will be some Buildings/Roads/Railways are going to be affected by the emergency volume of the UAS and are therefore considered within the risk assessment, Section 3.

##### 3.3.2.1.1 Population Assemblies

According to https://ec.europa.eu/regional_policy/mapapps/urban/degurba.html recommended by the EASA, there is either significant degree of industrialisation or urban clusters withing the area of operations, there is also a greater region of less industrialisation and Urban cluster than to Rural clusters. Figure 3.

Image 2: Population Assemblies

##### 3.3.2.1.2 Ground Infrastructure

The typical flight missions will involve flying over Road networks, Rail networks, Waterways, right of ways, foot tracks, buildings – Large, Medium and Small Powerlines, Bridges, planned routes/Missions will cross perpendicularly over the roads/rail’s networks and Waterways, all unnecessary areas will not be flown, and the most effective and efficient flight path will be chosen to cut down on the amount of time flown over any Infrastructure.

Image 3: Ground Infrastructure by County

##### 3.3.2.2 Airspace Class

The airspace within the volume of operations could be any one of the following.

- Different Types of Airspace in Ireland.

- Class C – Controlled

- Class G – Uncontrolled

- Military Airspace

- Restricted

- Danger

- Military Operating Area

- Prohibited Airspace

- Air Traffic Zone (ATZ)

- Temporary Restricted Area

- Temporary Segregated Area

##### 3.3.2.2.1 Airports, Airfields & Helipads

Your Company Name will be conducting Flight Operations in Rural and Urban within these areas there will be Registered and Unregistered Airfields/Airports/Helipads. We will cover all these in the SORA/PDRA. All RP will check the location for nearby Registered and Unregistered Airfields/Airports/Helipads, within a 5KM of the Survey site. They will make contact with the owner and inform them of our intended UAS operations. If the Owner does not respond, then the UAS Operation can still go ahead but the RP will try to make final contact prior to the Drone taking off.

Image 4: Example of Rural Aerodrome

#### 3.3.2.3 LTE Connectivity

Vodafone/Three are the chosen phone carrier. As shown in Figure 8, the 4G/LTE connectivity seems to be sufficient over the zones where the routes are planned. Nevertheless, the actual connectivity and reliability of the LTE network is to be tested during the scouting flights.

Image 5: LTE Connectivity Coverage for Vodafone

Image 6: LTE Connectivity Coverage for Three

#### 3.3.2.4 Weather

The following information is extracted from https://weatherspark.com for the average weather throughout the year.

According to Figure 9 through Figure 12, the expected weather conditions for the time of operations is as follows:

- Temperature: the average expected temperature is 10-16°C.

- Rain: the average daily rain probability is significantly height, 33-40%.

- Wind: the average wind is significantly high (11-14 mph).

- Humidity: the humidity comfort level is comfortable.

- Daylight: there are at least 12h of daylight.

Image 7: Temperature Average

Image 8: Daily Chance of Precipitation

Image 9: Average Wind Speed in Ireland

Image 10: Humidity Comfort Levels

Image 11: Hours of Daylight

### 3.3.3 Typical Flight Missions uncontrolled Airspace

The initial mission types are described in Table 7 and shown from Figure 13 through to 20. The mission types will either be 2d Mapping or 3D mapping. Manual flights will be conducted for Inspections and the event of the pilot taking control of the drone either during or after a Pre-planned flight mission. Linear Missions will be conducted in the same manner as 2D and 3D missions. The following deconfliction strategies apply:

- 2D/3D Flight Mission max height = 120m AGL Multi-Rotor

- Inspections Flight Missions max height = 120m Multi-Rotor

- Linear Flight Missions max height = 120m AGL Multi-Rotor

| N° | Mission Type Uncontrolled Airspace | Type of area over Flown | Max height [m] AGL |  | Airspace | Type |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Topographical (2D/3D) | Sparsely Populated Areas/Populated Areas | 120 |  | EINN | VLOS / Automated (Semi or manual) |
| 2 | Bridge/Roof/Façade (2D/3D) |  |  |  |  |  |
| 3 | Coastal Erosion/Defence/ (2D/3D) |  |  |  |  |  |
| 4 | Electrical Substations/Phone Mast/Electrical Power cables (2D/3D) |  |  |  |  |  |
| 5 | Waypoint Mission Manual |  |  |  |  |  |

Table 6: Flight Mission parameters

#### 3.3.3.1 Including new Flight Mission Parameters/workflows

In case of a new workflow for flight mission parameters that may need changing or be required, the following needs to be satisfied:

| N° | Mission Type Controlled/Restricted/Prohibited Airspace | Type of area over Flown | Max height [m] AGL | Airspace | Type |
| --- | --- | --- | --- | --- | --- |
| 1 | Topographical (2D/3D) | Sparsely Populated Areas/Populated Areas | 120 | EINN | VLOS / Automated (Semi or manual) |
| 2 | Bridge/Roof/Façade (2D/3D) |  |  |  |  |
| 3 | Coastal Erosion/Defence/ (2D/3D) |  |  |  |  |
| 4 | Electrical Substations/Phone Mast/Electrical Power cables (2D/3D) |  |  |  |  |
| 5 | Waypoint Missions Manual |  |  |  |  |

- The risk class of the flight mission parameters cannot compromise the level of compliance of the risk assessment in Section 3.

Table 7: Controlled airspace parameters

- The flight mission parameters must be contained in the approved area of operations, Section 3.1.2.

Image 12:Typical Flight Mission 2D

Image 13:Typical Flight Mission 3D

Image 14:Typical Linear Mission

Image 15:Typical Waypoint Mission - Uncontrolled Airspace

#### 3.3.3.2 Typical Flight Missions Controlled/Restricted/Prohibited Airspace

Image 16: Typical 2D Mission - Controlled Airspace

Image 17:Typical Linear Mission - Controlled Airspace

Image 18:Typical Waypoint Mission - Controlled Airspace

Image 19:Typical 3D Mission - Controlled Airspace

#### 3.1.3.4 Containment / Geofence

The operational volume, describe in Section 3, is contained by a polygonal geofence that runs parallel to the horizontal projection of the Mission area and closes the vertical dimension with a ceiling of 120meters. When Operating any Flight Mission within DJI App software DJI GO FLY the App has the ability to generate a GEO-Fence where the Drone will remain inside this GEO-Fence. The Standard Operating Procedure for Your Company Name is when the Flight Radius has been confirmed the RP will then input the horizontal distance and the Vertical Distance for the flight mission. See figure 21 & 22 for example.

Image 20: Containment/Geo-Fence/Boundary Box in DJI App

Image 21:Geo-Fence Parameters Diagram

#### 3.3.3.6 Geo-Fences around Airports – DJI FlySafe Geo-App.

The DJI FlySafe Geo-App can be used as a form of Safe Geofencing against certain Flight Emergencies, once GPS is still active. The DJI FlySafe parameter are proven to be very effective when operating within 5Km of the Airport. The DJI FlySafe Parameters will not let a DJI Drone take-off within a 5KM radius of the Main airports in Ireland, Dublin, Waterford, Cork, Kerry, Limerick, Knock. Sligo and Donegal without applying for an unlocking code through the DJI FlySafe Website. In order for a Drone operator to receive the Unlocking code the RP must fill-in an application form and procedure Proof that they have permission from SUAAIRSPACE@iaa.ie to conduct a Drone Operation within 5KM of the Airport. A screen grab of the operations permission from the terms and conditions email will be enough for DJI FlySafe Team. This and the Flight controller Serial Number must be in the Application in order for the RP to get the unlocking codes for the Drone to take-off in this environment. These unlocking codes will then be synced with the Aircraft and the Remote Controller to allow operations to go ahead. When applying for the unlocking codes the flight parameters must be added to the application and these are the parameters that the drone operation will take place under. At no stage will the DJI FlySafe Geo App allow the drone to fly outside of these parameters.

Under RED and BLUE Zones Unlocking Codes are required and can only be applied for through DJI FlySafe Website. There is Time Frames on these Codes. Your flight permission parameters from SUAAIRSPACE@iaa.ie with location, radius, flight height and date time group will be the DJI FlySafe Parameters. You will not be able to operate outside of these parameters. This is per application.

Image 22:DJI Flysafe Geo-Zones

Enhanced Warning & Warning Zones only require the Pilot to tick a box to acknowledge they are operating in these Zones. When operating in these Zones the RP will input the Mission Parameters as described in 3.1.3.4 Containment / Geofence.

### 3.3.4 Level of Involvement (LoI)

Although the RP is always in control of the UAS, the missions that are conducted with third party Flight Planning software or Third-Party Flight planning Apps, are semi - automated. The mission is uploaded to the flight controller with information on the navigation waypoints. Actions under normal, abnormal, and emergency situations will be conducted by the RP. However, the crew is present to watch for the safety of the flight and participates actively during the nonautomated parts of the operation if any.

The crew is involved in the following way:

- Remote Pilot will:

- Make sure Mission parameters are correct

- Checks Containment Fences / Geo-Fences are in place and correct.

- start, flies, interrupts and / or terminates the flight.

- monitors the flight, communicates with the VO (If Present), and executes commands under normal, abnormal, and emergency situations.

- takes manual control of the UAS under abnormal and / or emergency procedures in VLOS.

- Visual Observer If Present will: (If not these are added involvement for the RP)

- secures the site, handles the insertion of SD Card, and enhances the situation awareness of the RP during VLOS Operations.

- Become the crew member on site if the external workload exceeds the capabilities of the RP or on request from the RP.

The Remote Pilot must be aware of external particulars that may influence any aspect of the Drone Operation. The RP must have contingence plans in place. See OM Section 5. Contingency Procedures.

### 3.3.5 Normal Operation Strategy

The normal operation strategy consists of 4 phases through which the readiness of the crew and UAS is evaluated until the final roll out of full-scale operations.

#### 3.3.5.1 Flight Planning – Mission Brief.

During the Flight Planning phase, the crew gets familiar with the intended area that requires drone flights. The following goals are to be accomplished:

- Approval of Drone operation from Head of Drone Department.

- Mission Area / Working area to be defined.

- KML of Mission Area / Working Area to be confirmed and checked correct.

- Deliverables to be issued to Remote Pilot and processor.

- Drone Pilot and company Processor to discuss data collection of Mission parameters.

- The training of new crew members if not completed already.

- Test the readiness of the UASs in VLOS. (Only if UASs not flown in the last two weeks.)

- Brief issued to Remote Pilot that is going to be conducting the Operation/Project.

- Questions and Answers session from Drone Pilot to Chief Drone Pilot (if required)

#### 3.3.5.2 New Environment parameters

During the new environment parameters phase the crew continues its training and keeps on building up confidence in getting to know the system and the new environment from the Flight Planning phase or simply by being on site and conducting the following to ensure the new environment is safe:

- The new environment sites are visited in person either on the day they are required to be flown or if certain permission is requested to personally go and visit the site to conduct the following:

  - Identify local hazards.

  - Select a suitable landing area.

In case the new environment sites are not accessible due to COVID 19 restrictions, the RP will be requested to use all the Flight Planning features that is available to them to get a view and understanding of the new site. If the site is new to a particular Remote Pilot but not to the company and it has been flown previously then the previously Orthomosaic will be requested, and the new RP will familiarise themselves using this Orthomosaic in conjunction with Goggle Earth PRO. The new RP will touch base with the previous RP that has flown this Site and a Questions and Answers Session will be held between them.

- Important questions will be asked such as:

  - What are the communications on this Site?

  - What is the terrain like for take-off and landing on all the UAS Systems Your Company Name have to offer?

  - Range assessment of the UAS in the new operational environment.

  - Type of airspace and what usage of manned aircraft are operating in this airspace, whether it be Controlled or Uncontrolled.

  - Is the manned aircraft operating in the area of a?

    - Low Risk

    - Medium Risk

    - High Risk

    - Consider a TRA if Manned aircraft is of a High Risk

  - Type of ground risk is it densely populated or sparsely populated.

  - What other Air users are operating in this new environment?

    - Parachutists.

    - Ballooning.

    - Hand Gliding.

    - Para Gliding.

    - Drone Operators Commercial/Hobbyists.

    - Emergency services.

    - Is there Airspace restrictions from manned or unmanned aircrafts in place for this new environment.

    - Is it on the path of migrating Birds?

If needed and the request is approved the previously RP, that has flown this environment, will be present while the New RP to this Environment completes their flight missions.

#### 3.3.5.3 Flight Mission Validation.

During the Flight Mission Validation (FMV) phase the following with be checked:

- The RP are trained and validated, and the Operational Authorisation is current under new EU rules and regulations.

- The Mission parameters flight altitude are confirmed and are adjusted and validated to use within the Working Area and DJI App Parameters that the RP has imported.

- Certain Flights Missions can be validated if have been flown before.

- Almost all FMV will be carried out while Mission has been flown by RP, once points one and two are confirmed.

The duration of the validation phase depends on the background of the crew and complexity of the environment.

#### 3.3.5.4 Full-Scale Operations

Once all the training of staff members is complete and an Operational Authorisation under the Specific Category has been issued to Your Company Name then full-scale operations can begin. During this phase all Drone operations are carried out and approved by the Head of the Department who will inform the Chief Remote Pilot of any changes that maybe required.

### 3.3.6 Standard Operating Procedures

In this the following topics are presented:

- Operational information related to Normal Missions.

- Modifications or supplements to the existing procedures, roles or responsibilities stated in the OM.

#### 3.3.6.1 Normal Operating Procedures

The following normal procedures are defined in order to account for adaptations of those stated in the OM, (Section 4 – Normal Procedures).

#### 3.3.6.2 Flight Preparations

Modifications or addition of procedures during the flight preparations are presented below.

##### 3.3.6.2.1 TRA

If a Temporary Restricted Area is to be issued for any operation this will be submitted with a 90-day lead in time as requested by ANSP of the IAA. All Staff must know how a TRA operates prior to any Operations to be conducted under a TRA. A full operational briefing will be held if any Drone Operations are to be conducted under a TRA.

##### 3.3.6.2.2 NOTAMs

The RP shall use the following link of to obtain official NOTAM information on the area of operations:

https://www.iaa.ie/air-traffic-management/notam

NOTAM must only come from the official Irish Aviation Authority website.

##### 3.3.6.2.3 Flight Scheduling – with client.

The flights date time group will be discussed with the client at management level and are scheduled between Your Company Name and the client through telephonically or via email, preferably with enough lead in time for the Remote Pilots to plan and prepare all documentation before drone Operations commence.

The Make and Model of the Drone will be allotted to the Remote Pilot at this stage of the Operations. It will be their reasonability to have the correct equipment in working order and any permissions that might be required for the Project.

##### 3.3.6.2.4 Legal and Permissions

The RP must confirm that the following documents are present and valid:

- Operational Authorisation

- RP have their Open/Specific Category licence valid

- NOTAMs checked

- TRA (If Required)

- Insurances of the Your Company Name

- If Client requests a RAMS, H&S of Your Company Name to complete and send on to client/RP/Chief Remote Pilot.

#### 3.3.6.3 Pre-Flight

Modifications or addition of procedures during the pre-flight are shown below.

##### 3.3.6.3.1 Weather forecast

During the planning phase of the flight the RP will check weather apps for localised weather. The following Apps to be checked in particular for the following limitations:

Light, windspeed, Cloud base, Visibility, Temperature, Wind gust, difference in wind at 10m, 50m, 100m, Rain radar and forecasted.

- Met Eireann

- Ventusky

- Windy

- Google Weather

The boundary parameters are to be defined as follows:

- max wind speed on take-off = 10 m/s

- max wind speed in cruise = 10 m/s

- max horizontal gust speed = 10 m/s

- max probability of rain prediction = Not to be flown in Rain

- Cloud base = 120m/400ft

• 	if visibility is less than = 800m. (NO GO in VLOS)

#### 3.3.6.4 Flight Execution

Modifications or addition of procedures during the flight execution are presented below.

##### 3.3.6.4.1 Weather monitoring

The RP shall use Windy/Ventusky or Met Eireann to monitor the rain radar.

##### 3.3.6.4.2 DJI App/eMotion Software

The RP shall use the DJI Sense and detect system in the App to spot and monitor other aircraft during the flight operations.

#### 3.3.6.5 Contingency and Emergency Procedures

Besides the following, there are no modifications to those stated in the OM.

##### 3.3.6.5.1 Contingency procedures

As laid out in Section 5.0 of this manual

Image 23:Loss of Connection with DJI App Software

3.3.6.5.2 Loss of Connection with the DJI Ap

Loss of Connection with DJI App

Time Without connection

Reconnected

NO Reconnection

Continue with Mission

App will Trigger Return to Home. RP will also trigger

#### 3.3.6.6 Occurrence Reporting

As laid out in Section 10 of this manual

#### 3.3.6.7 Operational Limits

##### 3.3.6.7.1 Altitude and Height

The Flight Missions are planned at different heights between the 0 and 120 m AGL. This height limitation may not be always satisfied due to different characteristics of the Project, if there is a requirement to conduct operations above 120meters AGL Permission will be requested through the IAA.

##### 3.3.6.7.2 Times of Operations

The operations shall only take place during the day. Although the UAS is equipped with navigations lights and does not require visual aids for navigation, the procedures stated in this section are conceived for operations during the day light only.

##### 3.3.6.7.3 Weather

The following weather limitations are applicable:

- Wind: When conducting any flight mission, the max limitation for wind is 10m/s on both Multi-Rotor Drone operations.

- Rain: No flight shall take place if there is predicted rain along any of the flight missions’ operational areas or during the time of the mission. If UAS is in a flight mission and downpour happens RP will Return to Home with the aircraft. RTH will not be triggered. RP will fly the drone back under control if it’s the Multi-Rotor.

- Visibility: minimum horizontal visibility = 800m.

- Could Clearance: with a minimum ceiling of 120m.

##### 3.3.6.7.4 Crew

The minimum number of crew members is different depending on the nature of the operation:

- For Multi-Rotor flights that are deemed Low or Medium Risk operations:

  - 1x RP

- For Multi-Rotor flights that are considered High Risk operations

  - 2 X RPs

##### 3.3.6.7.5 Telemetry Timeout

If while conducting Drone Operations and the RP Telemetry goes blank and no live feed is being reported back to the Remote Controller, if the RP cannot get a positive connection, they will terminate the flight and trigger the Return Home Function. The time frame to make this decision will be less than 30 seconds. The RP will reboot the Remote Controller Tablet/SmartController/iPad/Phone it will take longer than 30 seconds to reboot, the RP will monitor the Aircraft and if it starts to go off mission flight lines, he will trigger RTH Function immediately.

#### 3.3.6.8 Emergency Response Plan

See Section 7 for further information on the Emergency Response Plan

| N° | Station | Telephone | Contact Person |  |  |
| --- | --- | --- | --- | --- | --- |
| 1 | Police | 112 or 999 |  |  | - |
| 2 | Fire Fighter | 112 or 999 |  |  | - |
| 3 | Ambulance | 112 or 999 |  |  | - |
| 4 | ATC | Dublin – 01 8144601 Waterford – 051 850341 Sligo – 071 9168461 Baldonnell – 01 4037514 |  | Cork – 021 4237853 Shannon – 061 471868 Weston – 01 6217301 |  |
| 5 | IAA | +353 86 605 1122 | Lou Fine |  |  |

Table 8: Emergency Phone Contacts

The fire, ambulance and police services in Ireland are all contactable via the above numbers. When calling emergency services, you will need to provide the following:

- The exact address of the incident or emergency and/or any noticeable landmarks nearby.

- Directions to the scene of the emergency.

- The telephone number you are calling from.

- Details on the incident itself, the number of persons involved, the description, any visible injuries and knowledge of any pre-existing medical conditions.

Try to stay calm and listen to the call taker’s instructions.

It is also important to keep your own phone on as the emergency service may need to contact you for further information. The RP shall use Table 11 to write down relevant information prior the call:

## 3.4 SORA - Specific Operating Risk Assessment for Risk analysis and methods for reduction of identified risks

The SORA analysis is performed using the information provided in SORA EU 2019/947. To facilitate the comprehension of this Section, the reader is recommended to follow the guidelines of the SORA analysis stated in EU 2019/947(Figure 25 Table 4).

Image 24: Determination of Robustness Level SORA EU 2019/947 (Table 4)

The robustness of the proposed mitigations is determined using Figure 25.

## 3.4.1 Applicability

As stated in the SORA (see Figure 25 Table 4):

- The methodology presented in this document is aimed at evaluating the safety risks involved with the operation of UAS of any class, size, or type of operation.

- The safety risks associated with collisions between UA and manned aircraft are in the scope of the methodology. The risk of a collision between two UA or between a UAS and a UAS carrying people will be addressed in future revisions of the document.

- In the event of a mishap, the carriage of people or payloads on board the UAS (e.g., Weapons) that present additional hazards is explicitly excluded from the scope of this methodology.

- Security aspects are excluded from the applicability of this methodology when they are not limited to those confined by the airworthiness of the systems (e.g., the aspects relevant to protection from unlawful electromagnetic interference.)

- Privacy and financial aspects are excluded from the applicability of this methodology.

- The SORA can be used to support waiving the regulatory requirements applicable to the operation if it can be demonstrated that the operation can be conducted with an acceptable level of safety.

- In addition to performing a SORA in accordance with the UAS Regulation, the UAS operator must also ensure compliance with all the other regulatory requirements applicable to the operation that are not necessarily addressed by the SORA.

## 3.4.1.2 Company RAMS (Risk assessments and Method Statements).

Company policy is that all drone operations cannot be conducted without a Company RAMS to be completed prior to operation. These RAMS must be submitted to Safety Manager for review and must be submitted to client prior arrival on site for their approval. Please see company Document RAMS ver1.2 this is the document that all Drone Parties weather Third party or company Drone operations must fill in before any drone operations.

## 3.4.2 Ground Risk

The ground risk estimation is the determination of risk of a person being struck by the AUV in case of loss of control with a reasonable assumption of safety.

The GRC determination is summarized as:

- Based on the nature of the operation the IGRC = 4

- The level of robustness of the proposed mitigations enables a reduction on the IGRC of -1 points.

- The mitigated GRC is = 3.

#### 3.4.2.1 Determination of Intrinsic Ground Risk

The kinetic energy of the Mavic 2 Enterprise Advanced is estimated as follows:

𝐸𝑐 =𝑚𝑣2

With m = MTOW = 907g and v = airspeed = 5m/s, Ec = 11.3375 J. Therefore the 1st column (<700J) in Figure 26 is chosen.

- The operational scenario is a VLOS flight over sparsely populated environment.

Combining both the operational scenario and the kinetic energy, the intrinsic UAS ground risk is:

IGRC = 2

- The operational scenario is a VLOS flight overpopulated environment.

Combining both the operational scenario and the kinetic energy, the intrinsic UAS ground risk is:

IGRC = 4

Overall IGRC

The overall IGRC for the different types of Operations.

VLOS in a Sparsely Populated environment is 4 for all drones and for Operations in VLOS in a populated environment is 2 for all drones.

Figure 25: Intrinsic Ground Risk Class (SORA EU 2019/947 Figure 25 Table 4)

#### 3.4.2.4.1 Determination of Mitigated Ground Risk

Figure 26:Mitigations for final GRC Determination (SORA EU 2019/947 Figure 25 Table 4)

The summary of the robustness determinations is presented in Table 13.

| Mitigation Sequence | Mitigations for Ground Risk | Robustness | Correction Factor |
| --- | --- | --- | --- |
| 1 | M1-Strategic mitigations | Medium | -2 |
| 2 | M2- Effects of ground impact reduction | Low | 0 |
| 3 | M3- ERP | Low | 1 |

Table 9: Ground Risk Mitigation Robustness

The mitigated ground risk category is calculated as the addition of the intrinsic ground risk and the corrections factors of Table 13.

With IGRC = 2, M1 = -2, M2 = 0 and M3 = 1, the mitigated risk category is:

MGRC = 1

#### 3.4.2.4.2 M1-Reduced Affected People

The following mitigations are trying to reduce the number of people at risk:

Requirement 1 Definition of the ground risk buffer The ground risk buffer considers:

- Improbable single malfunctions which could lead to an operation outside the volume, e.g., firmware bugs, power plant failures, GNSS glitches, pilot error, etc.

- Meteorological conditions, e.g., drifting during the transition due to cross wind.

- UAS latencies: the LTE latencies are considered, however a containment geofence is put in place.

- UAS behaviour when activating the technical containment: in case the UAS crosses the geofence, it would require another 100m to turn back and follow the RTH route. This extra 100m are not considered in the risk buffer.

Since the integrity of the mitigations above can only be partially satisfied the level of integrity is considered as:

Requirement 1 level of integrity: Low

The following evidence supports assurance of the mitigations above:

- Based on internal incident and accident investigations, single malfunctions lead to a crash inside the 1:1 distance.

- The study of previous operations reveals that the UAS can drift up to 6meters in Flight Plan due to cross winds.

- As shown in the respective interfaces: the LTE latencies are generally below 1 second.

Since the assurance of the mitigations above can be supported with tests and evidence, the level of assurance is considered as:

Requirement 1 level of assurance: Medium

Combining both the level of integrity and the assurance, the robustness of the requirement 1 is:

Requirement 1 level of robustness: Low

Requirement 2 Evaluation of people at risk – for the Mavic 2 PRO Drone.

- A UAS of MTOW 907g and Ideal operating speed of 5m/s

- The operation is conducted in a sparsely environment and populated area.

Additionally,

- The risk related to isolated houses and roads that lie inside the contingency volume are addressed in the risk matrix assessment.

- Roads are crossed as perpendicularly as possible to reduce the exposure time.

- The take-off and landing areas are watched after by RP to secure the area and make sure no person enters the danger zone.

- RP to identify all ELZ throughout the flight.

- If need to Terminate flight do so with RTH Function firstly, then go to Forced Landing Procedure.

Since the integrity of the mitigations above is fully satisfied the level of integrity is considered as:

Requirement 2 level of integrity: High

The following evidence supports assurance of the mitigations above:

- The population density data use for the evaluation of the ground risk is obtained from two official static sources: ec.europa.eu Figure 2 and Figure 3.

Based on the evidence provided above the level of assurance for criterium 2 is:

Requirement 2 level of assurance: High

Combining both the level of integrity and the assurance, the robustness of the requirement 2 is:

Requirement 2 level robustness: High

Using the information provided in Figure 25, the resultant robustness of the criterium 1 and 2 yields:

M1 Robustness: Medium

#### 3.4.2.4.3 M2- Effects of ground impact reduction

Remote Pilots (RP) are highly trained and can identify an incident in the air that as a result could lead to a ground risk. The RP will land the aircraft at a pre-selected Emergency Landing Zone. These are part of the flight planning phase, selected by the RP.

- Construction Sites – All Personal working on a Construction site wear PPE which consists of a hard hat, hi-vis vest/jacket, steel toe cap boots, long sleeves and eye protection. Some sites require gloves. The ground risk is reduced even further as no public person are allowed access to a construction site. As these are cordoned off areas. These mitigation factors reduce the Injury in the event of a loss of control during a flight plan.

- Quarries - All Personal working in a Quarries wear PPE which consists of a hard hat, hi-vis vest/jacket, steel toe cap boots, long sleeves and eye protection, and gloves. Other workers are in moving vehicles and therefor are protected in the event of loss of control of the mission. The ground risk is reduced even further as no public person are allowed on to a quarry unless they are inducted. These mitigation factors reduce the Injury in the event of a loss of control during a flight plan.

- Rail Networks – no unofficial persons are allowed on any rail network without prior knowledge and arrangement with Irish Rail. When operating in and around Rail Networks. This type of operation is for conducting Rail Bridge inspections.

- Road Networks – flight plans for Road Network operations will be designed in a manner where the drone will be flying over the road at the shortest distance. The injury to life is reduced as persons are inside the car in the event of loss of control of the mission. When an incident happens where a drone lands on a car in traffic the Emergency Response Plan will be activated by the RP. Emergency Landing Zones shall be selected along the flight path and used in the event of an emergency.

#### 3.4.2.4.4 M3- Emergency Response Plan

The ERP presented in section 7, complies with the following conditions:

- It is suitable for the situation, since the operational area of this project does not pose hazards that are not included in the ERP. Additionally, it includes important telephone numbers of relevant stations such as ATC, and emergency services.

- It limits the scaling effects of an accident, e.g., fire or mid-air collision, by quickly communicating the nature of the situation and last known location to the relevant station.

- The criteria to identify an emergency situation is defined according to SORA EU 2019/947

- the outcome of the situation relies highly on providence; or

- It could not be handled by a contingency or emergency procedure; or

- when there is a grave and imminent danger of fatalities.

- It is of practical use since it is schematic and simple.

- It describes the role of the crew members.

Based on the above, the level of integrity of the ERP is considered as:

M3 level of integrity: Medium

The level of assurance of the conditions stated above is evaluated:

Requirement 1 Procedures

- The procedures are not validated against the means of compliance of the competent authority.

- The adequacy of the procedures and checklists is declared.

Based on the above the level of assurance of requirement 1 is:

Requirement 1 level of assurance: Low

Requirement 2 Training

- The ERP is included in the training syllabus, see Section 4.5.

- A record of the training completions is established and kept up to date.

Based on the above the level of assurance of requirement 2 is:

Requirement 2 level of assurance: Medium

Using the information provided in Figure 25, the resultant assurance of the criterium 1 and 2 yields:

M3 level of assurance: Low

Using the information provided in Figure 25, the resultant robustness of the M3 yields:

M3 level of robustness: Low

## 3.4.3 Air Risk

The air risk estimation is the determination of risk of mid-air collision with other manned aircraft with a reasonable assumption of safety.

Summarizing the ARC determination shown in the following Sections:

- The initial ARC is conservatively considered as ARC-c since the airfields and helipad are included as if they had significant air traffic.

- The strategic and tactical mitigations proposed comply partially with both the ARC-b and ARC-c levels of robustness.

- The initial ARC is therefore reduced to ARC-b, and this would be the only assumable ARC until the robustness of the DAA system is documented on the field.

The Air Risk is determined by following the flowchart

#### 3.4.3.1 Determination of Initial Air Risk Class

The operation as described in 3.1.2, takes place in:

- Below 400ft AGL

- Controlled airspace with Permission.

- Temporary Restricted Areas with Permission.

- Air Traffic Zones with Permission.

- Restricted/Prohibited Airspace.

- Airport/heliport environments.

- In airspace class G over rural area.

Using the diagram shown in Figure 27, the operational environment is:

Initial aircraft encounter category: 3

Initial generalised density rating: AEC 2

Initial ARC: ARC-c

Figure 27:Initial ARC Assignment (SORA EU 2019/947 (Figure 25 Table 4))

#### 3.4.3.2 Lowering the Initial ARC

The reduction of initial ARC risk using Figure 27 is justified with the mitigations presented in Section 3.4.3. Based on the strategic and tactical mitigations, the initial ARC is reduced one level to:

Residual ARC: ARC-b

Reduced density rating: AEC 1

Figure 28: Lowering the Initial ARC (SORA EU 2019/947 (Figure 25 Table 4))

#### 3.4.3.2.1 Strategic Mitigation

The following strategic mitigations are presented to lower the initial ARC.

Operational Restriction

- Geographical Boundary: the routes are planned so that the minimum distance to airports / heliports is at least 1 km when operating in Controlled Airspace.

- Time Limitations: No mitigation of this type is proposed.

- Time of Exposure: The time of exposure to airport/heliport environments occurs for small time windows of the mission. ATC can have a Time constraint on our Operations once inside Controlled Airspace with approval.

Common Structures and Rules

- The Mavic 2 PRO once flying in DJI Pilot PE App it is equipped with electronic cooperative system (DAA), and navigation and anti-collision lighting.

- Phantom Series have Green and Red LEDs front and Back.

- NOTAMS are read during the flight-preparations phase.

- ATC is contacted during the flight-preparations phase to obtain information about other manned aircraft operating on our area, if operating in high volume of Manned aircraft.

- ATC is contacted during the pre-flight and the post flight to communicate the beginning and end of the flight operations.

- App Boundary Containment/Geofence limits are used to keep the Drone within the Flight Geographical Zone.

#### 3.4.3.2.2 Tactical Mitigation Performance Requirement - TMPR

Tactical mitigations are those applied after take-off to reduce the rate of collision by modifying the geometry and/or dynamics if the aircraft in conflict based on real-time aircraft conflict information. A Tactical Mitigation could be a piece of hardware installed in the aircraft.

For this operation, the following tactical mitigations are proposed:

- DAA: based on a combination of ground-based and air-based system with pilot actions in the loop.

- All flights are VLOS this is a considered to be an acceptable tactical mitigation for collision risk for all ARC levels.

#### 3.4.3.2.3 TMPR Qualitative Requirement

TMPR qualitative requirement from both ARC-b and ARC-c are partially satisfied:

Detect

- The air traffic detection system should detect 90% of the manned aircraft in the detection volume 15km away of the UAS.

- The pilot is aware of the air traffic via the DAA and other web-based real time aircraft tracking systems.

- The Use of Flight Radar 24 App is an additional backup for the DAA system that’s in the DJI Pilot PE App.

Decide

The RP is trained to execute evasive commands as shown in the abnormal and emergency procedures.

Command

- The primary LTE C2 link does not exceed the latency threshold of 3 seconds.

Execute

The following commands can be executed depending on the nature of the situation:

- Hold the mission loitering about a point.

- Hold and descent / ascent to a safe altitude in less than 1 min.

- Go-To a different location.

- Land on the spot. (Direct landing feature – )

- Activate Return to Home Function.

These Actions can be carried out for Multi-Rotor &  Drones.

Feedback Loop

The DAA system should have the following characteristics:

- The refreshing rate of the system is around the one second.

- The latency of the system is normally around 3 seconds. This is however dependent on the mobile phone network.

- The system should detect 90% of the manned aircraft in the detection volume.

- The range of detectability should reach at least the 15km.

Note: the statements above referring to the feedback loop are neither proven nor documented by the system provider and have been tested on the field.

#### Robustness

The DAA system satisfies conditions of both “Low and Medium” levels of integrity and assurance. Based on Your Company Name past experience, it is difficult to define with certainty the exact level of robustness, which should currently be between “Low and Medium”. In order to be conservative, the lowest of both is chosen, expecting to prove during the project that the level of robustness is higher. The accepted ARC according to the level of robustness is:

DAA Level of Robustness: Low (ARC-b)

Level of Integrity

It is expected that the loss of function of the performance of the tactical mitigation system lies between the 1E-2 and the 1E-3. There is no quantitative analysis to sustain the statement above, but it is based on 1000h of VLOS experience. Therefore, the level of integrity of the DAA system is according to Figure 30:

DAA Level of Integrity: Low (ARC-b)

Level of Assurance

Your Company Name has already used this DAA system once in the past, being able to detect more than 90% of the manned aircraft and avoid collisions with manned aircraft. Your Company Name also use a third-party app in conjunction with the DAA system. The third-party app is ‘Flightradar24’

DAA Level of Assurance: Low (ARC-b)

Figure 29: TMPR Integrity and Assurance Objectives (SORA EU 2019/947 (Figure 25 Table 4))

## 3.4.4 SAIL

Using the information provided in Figure 26, with a final GRC = 3 & a residual ARC-b  the SAIL is determined as:

SAIL = II

Figure 30:SAIL Determination (SORA EU 2019/947 (Figure 25 Table 4))

## 3.4.5 OSOs

The Operation Safety Objectives are list below in two tables. There is 24 in total. The SAIL number is a guideline on what presently the mitigation level is. Weather it’s a Low (L), Medium (M), High (H), Optional (O). We will now mitigate as many of these OSO’s to show this operation can be conducted Safely. In some OSO’s if we can achieve the requirement and succeed it we will reflect this in Table 14: summary of OSO Analysis.

|  | Construction Sites |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| OSO #25 | Operational procedures are defined, validated and adhered to | L | M | H | H | H | H |
| OSO #26 | The Remote Pilot is trained to identify critical environmental conditions or changes and to avoid them | L | M | H | H | H | H |
| OSO #27 | Environmental conditions for safe operations are defined, measurable and adhered to | L | L | M | H | H | H |
| OSO #28 | Flight Geography and ground risk adhered to | L | M | H | H | H | H |
| OSO #29 | Flight Geography and Air Risk adhered to | L | M | H | H | H | H |
|  | Quarry Sites |  |  |  |  |  |  |
| OSO #30 | Flight Geography and ground risk adhered to | L | M | H | H | H | H |
| OSO #31 | Flight Geography and Air Risk adhered to | L | M | H | H | H | H |
| OSO #32 | The Remote Pilot is trained to identify critical changes to environmental conditions, Dust Clouds, Blast areas | L | M | H | H | H | H |

| OSO | Integrity | Assurance | Robustness | Objective | Satisfied | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Insert O,L,M or H under the correct heading |  |  |  |  | Y/N | Add comments if need be. |
| 1 | - | - | - | L |  |  |
| 2 | - | - | - | O |  | The EASA has defined manufacturing standards to comply with but not to be implemented until Jan 2023 |
| 3 | - | - | - | L |  |  |
| 4 | - | - | - | O |  | The EASA has defined manufacturing standards to comply with but not to be implemented until Jan 2023 |
| 5 | - | - | - | O |  | The Drone Manufacturer have a recorded the testing and research into it safety features of the drone. |
| 6 | - | - | - | L |  | The supporting evidence is in progress |
| 7 | - | - | - | L |  |  |
| 8 | - | - | - | M |  | Human error can still lead to accidents, and The crew does not receive specific crew resource management (CRM) |
| 9 | - | - | - | L |  |  |
| 10 | - | - | - | L |  | Integrity and assurance levels are met due to the experience and training of Pilots. |
| 11 | - | - | - | M |  | Human error can still lead to accidents, and The crew does not receive specific crew resource management (CRM) |
| 12 | - | - | - | L |  | Integrity and assurance levels are met due to the experience and training of Pilots. |
| 13 | - | - | - | L |  |  |
| 14 | - | - | - | M |  | Human error can still lead to accidents, and The crew does not receive specific crew resource management (CRM) |
| 15 | - | - | - | L |  |  |
| 16 | - | - | - | L |  |  |
| 17 | - | - | - | L |  |  |
| 18 | - | - | - | O |  |  |
| 19 | - | - | - | O |  |  |
| 20 | - | - | - | L |  |  |
| 21 | - | - | - | M |  | Human error can still lead to accidents, and The crew does not receive specific crew resource management (CRM) |
| 22 | - | - | - | L |  |  |
| 23 | - | - | - | L |  |  |
| 24 | - | - | - | O |  |  |
| 25 | - | - | - | M |  |  |
| 26 | - | - | - | M |  |  |
| 27 | - | - | - | L |  |  |
| 28 | - | - | - | M |  |  |
| 29 | - | - | - | M |  |  |
| 30 | - | - | - | M |  |  |
| 31 | - | - | - | M |  |  |
| 32 | - | - | - | M |  |  |
| Point to Note: a change in color in Objective column means Overall Level Of Robustness has been superseded. |  |  |  |  |  |  |

Table 10: Summary of the OSO Analysis

#### 3.4.5.1 OSO01

Category: Technical issue with the UAS

Topic: Ensure the UAS operator is competent and/or proven

#### 3.4.5.1.1 Level of Integrity

- Your Company Name is truly knowledgeable of the UAS being used.

- There are relevant operational procedures in place (see Section 4.1)

  - Checklists

  - Maintenance

  - Training

  - Responsible

  - Associated duties

- There is a method to identify and assess the risk associated to the operation, based on the design of the operation made by the FOM and the flight preparations risk management sated.

The current level of integrity is:

Level of integrity OSO01: LOW/MEDIUM/HIGH

#### 3.4.5.1.2 Level of assurance

- The elements detailed in the level of integrity are addressed in this ConOPS The current level of assurance is:

Level of assurance OSO01: LOW/MEDIUM/HIGH

#### 3.4.5.1.3 Level of Robustness

Based on the above the current level of robustness is:

Level of robustness OSO01: LOW/MEDIUM/HIGH

#### 3.4.5.2 OSO02

Category: Technical issue with the UAS

Topic: UAS manufactured by competent and/or proven entity

#### 3.4.5.2.1 Level of Integrity

DJI manufacturing procedures cover:

Specification of materials, suitability and durability of the materials, and  the processes necessary to allow for repeatability in manufacturing, and conformity within acceptable tolerances.

The current level of integrity is:

Level of integrity OSO02: LOW/MEDIUM/HIGH

#### 3.4.5.2.2 Level of assurance

EASA have published Manufacturer’s requirements for different types of CE marking requirements under EU Delegated Regulation 2019/945. The CE marking of Drones will not happen until Jan 2023. Because of this our Level of assurance is Low.

The current level of assurance is:

Level of assurance OSO02: LOW/MEDIUM/HIGH

#### 3.4.5.2.1.3 Level of Robustness

Based on the above the current level of robustness is:

Level of robustness OSO02: LOW/MEDIUM/HIGH

#### 3.4.5.3 OSO03

Category: Technical issue with the UAS

Topic: UAS maintained by competent and/or proven entity

3.4.5.3.1 Level of Integrity

Your Company Name complies with:

The definition of maintenance instructions aligned with the instructions and requirements of the design department. The maintenance staff is competent and has been authorized to conduct UAS maintenance. The maintenance staff uses the UAS maintenance instructions while performing maintenance. Scheduled maintenance of each UAS is organized and in accordance with a maintenance program.

The maintenance log is used to record all the maintenance conducted on the UAS including releases.

The current level of integrity is:

Level of integrity OSO03: LOW/MEDIUM/HIGH

#### 3.4.5.3.2 Level of assurance

Based on the level of assurances of both requirements shown below the level of assurance is:

Level of assurance OSO03: LOW/MEDIUM/HIGH

Requirement 1

- The maintenance instructions are documented, see AMM in Table 4.

- The maintenance conducted on the UAS is recorded in a maintenance log system.

- A list of the maintenance staff authorized to conduct maintenance is established and kept up to date.

Level of assurance Requirement 1: LOW/MEDIUM/HIGH

Requirement 2

A record of all the relevant qualifications, experience and/or training completed by the maintenance staff is established and kept up to date.

Level of assurance Requirement 2: LOW/MEDIUM/HIGH

#### 3.4.5.3.3 Level of Robustness

Based on the above the current level of robustness is:

Level of robustness OSO03: LOW/MEDIUM/HIGH

#### 3.4.5.4 OSO04

Category: Technical issue with the UAS

Topic: UAS developed to authority recognized design standards

#### 3.4.5.4.1 Level of Integrity

The EASA have release Design Standards for Drones to Operate in the European Union but have allowed time for Drone Manufacturers to have these Design Standards in place in the form of a CE Marking. The timeline is January 1st 2023. At present no requirement from EASA to implement the Design Standards.

Level of integrity OSO04: LOW/MEDIUM/HIGH

#### 3.4.5.4.2 Level of assurance

EASA have published Manufacturer’s requirements for different types of CE marking requirements under EU Delegated Regulation 2019/945. The CE marking of Drones will not happen until Jan 2023. Because of this our Level of assurance is Low.

Level of assurance OSO04: LOW/MEDIUM/HIGH

#### 3.4.5.4.3 Level of Robustness

Level of Robustness OSO04: LOW/MEDIUM/HIGH

#### 3.4.5.5 OSO05

Category: Technical issue with the UAS

Topic: UAS is designed considering system safety and reliability

#### 3.4.5.5.1 Level of Integrity

Upward, Downward and Sideward Avoidance Sensors with audible tones to warn pilot of close objects. Low battery warnings and Critical battery warnings with audible tones to warn pilot of error. At low battery warning the aircraft is designed to active the “Return to Home function”. The Software allows the pilot to create a Geo-Fence Boundary Box that maintains the drone inside it. This feature can be changed according to Mission Parameters and from mission to mission.

Level of integrity OSO05: LOW/MEDIUM/HIGH

#### 3.4.5.5.2 Level of assurance

The safety aspects of these aircraft are very similar as they are manufactured by the same company. The Safety features have been tested and all Pilots are fully trained on how to recognises these Issues and respond correctly. The training that is required is covered under the Emergency procedures of the Internal Drone training course. This is recorded and all students receive a Course completion on successfully passing the course.

Level of assurance OSO05: LOW/MEDIUM/HIGH

#### 3.4.5.5.3 Level of Robustness

Level of robustness OSO05: LOW/MEDIUM/HIGH

#### 3.4.5.6 OSO06

Category: Technical issue with the UAS

Topic: C3 link performance is appropriate for the operation

#### 3.4.5.6 Level of Integrity

Your Company Name states that the performance, RF spectrum usage and environmental conditions for C3 links are adequate to safely conduct the intended operation, and

The systems of the UAS use mechanism to protect itself against interferences.

The RP has the means to continuously monitor the C3 performance and ensures that the performance continues to meet the operational requirements.

Based on the above, the low level of integrity is partially satisfied.

Level of integrity OSO06: LOW/MEDIUM/HIGH

#### 3.4.5.6.2 Level of assurance

The LTE telemetry link quality is shown to the Remote Pilot in the Drone live view on tablet/Phone/Laptop.

The RC signal strength is shown under the Signal Strength in the Live App.

Your Company Name declares that this level of integrity has been achieved. Supporting evidence is on progress.

Considering the above, the level of assurance is:

Level of assurance OSO06: LOW/MEDIUM/HIGH

#### 3.4.5.6.3 Current Level of Robustness

Based on the above the level of robustness is:

Level of robustness OSO06: LOW/MEDIUM/HIGH

#### 3.4.5.7 OSO07

Category: Technical issue with the UAS

Topic: Inspection of the UAS (product inspection) to ensure consistency with the ConOps

#### 3.4.5.7.1 Level of Integrity

The remote crew ensures that the UAS is in a condition for safe operation and conforms to the approved ConOps. The level of integrity is defined by the level of assurance.

Level of integrity OSO07: LOW/MEDIUM/HIGH

#### 3.4.5.7.2 Level of assurance

Based on the level of assurance of the requirements below:

Level of assurance OSO07: LOW/MEDIUM/HIGH

Requirement 1

- Product inspection is documented in the journey log and accounts for the manufacturer’s recommendations if available.

- The product inspection is documented using checklists, see Appendix A.

Level of assurance Requirement 1: LOW/MEDIUM/HIGH

Requirement 2

- A training syllabus including a product inspection procedure is available, see Section 3.7.

- The UAS operator provides competency-based, theoretical, and practical training.

Level of assurance Requirement 2: LOW/MEDIUM/HIGH

#### 3.4.5.7.3 Current Level of Robustness

Based on the above the current level of robustness is:

Level of robustness OSO07: Medium

#### 3.4.5.8 OSO08

Category: Technical issue with the UAS

Topic: Operational procedures are defined, validated, & adhered to

OSO 08 covers also OSOs: 11, 14 and 21.

#### 3.4.5.8.1 Level of Integrity

Based on the summation of Requirements below the level of integrity is:

Level of integrity OSO08: Medium

Requirement 1 procedure definition

- Operational procedures appropriate for the proposed operation are defined and, as a minimum, cover the following elements:

- Flight planning.

- Pre- and post-flight inspections see Section 4.1

- Procedures to evaluate the environmental conditions before and during the mission (i.e., real-time evaluation).

- Procedures to cope with unexpected adverse operating conditions see Section 7

- Normal procedures see Section 4.

- Contingency procedures (to cope with abnormal situations see Section 5).

- Emergency procedures (to cope with emergency situations see Section 6).

- Occurrence reporting procedures see Section 10; and

- The limitations of the external systems supporting UAS operation are defined in an OM.

Considering the above:

Level of integrity Requirement 1: LOW/MEDIUM/HIGH

Requirement 2 procedure complexity

- VLOS contingency and emergency procedures do not require manual control of the RP, but the activation of commands.

- The operational procedures are designed seeking simplicity and safety. Considering the above:

Level of integrity Requirement 2: LOW/MEDIUM/HIGH

Requirement 3 consideration of potential human error

- The operation is designed to be conducted by the minimum number of crew members, among which the distribution and assignment of tasks, and

- An internal checklist ensures the crew are adequately performing their assigned tasks.

The procedures take human error in consideration, by means of:

- 4 eye check during mission planning. o Use of checklists during the different flight phases.

- Practice of abnormal and emergency situations.

- Aeronautical decision making is included in the training syllabus.

- The training and procedures are designed so that the RP works together with the rest of the crew reducing errors, avoiding stress, and increasing efficiency. However,

- Human error can still lead to accidents, and

- The crew does not receive specific crew resource management (CRM)

Considering the above:

Level of integrity Requirement 3: LOW/MEDIUM/HIGH

#### 3.4.5.8.2 Level of assurance

- The operational procedures have been designed, improved, and validated based on Your Company Name experience in more than 1000h of VLOS operations.

- The adequacy of the contingency and emergency procedures is proven though dedicated flight testing.

- The flight tests performed to validate the procedures and checklists cover the complete flight envelope.

- The procedures, checklists, flight tests and simulations are validated by a competent third party.

Based on the above the level of assurance is:

Level of Assurance OSO08: LOW/MEDIUM/HIGH

#### 3.4.5.8.3 Current Level of Robustness

Based on the above stated levels of assurance and integrity the current level of robustness is:

Level of robustness OSO08: LOW/MEDIUM/HIGH

#### 3.4.5.9 OSO09

Category: Technical issue with the UAS

Topic: Remote crew trained and current and able to control the abnormal situation

#### 3.4.5.9.1 Level of Integrity

The RPs involved in the operation have competency-based, theoretical, and practical training adequate for the operation covering:

- the UAS regulation. 		   (i) Data Protection

- airspace operating principles.	   (j) UAS Performance

- airmanship and aviation safety;   (k)Technical & Operational Mitigations for Air risks

- human performance limitations.  (l)Technical &Ops Mitigations for Ground risks

- meteorology. 			   (m) SORA & PDRA – STS-01

- navigation/charts.

- operating procedures.

- Flight Exam by IAA Examiner

Level of integrity OSO09: LOW/MEDIUM/HIGH

#### 3.4.5.9.2 Level of assurance

- Point (h) of the items exposed above, is included in the syllabus of the theoretical and practical training of Your Company Name, see Section 2.6.

- The training syllabus of Your Company Name and the crew competencies are verified by a competent third party, Survey Drones Ireland who are a Recognised Entity (DUTO)

- Points (a) through (m) are covered in the syllabus of a certified ground and flight training where the pilots obtain their Remote Pilot Licence.

The level of assurance is therefore:

Level of assurance OSO09: LOW/MEDIUM/HIGH

#### 3.4.5.9.3 Current Level of Robustness

Based on the above determined levels of integrity and assurance:

Level of robustness OSO09: LOW/MEDIUM/HIGH

#### 3.4.5.9.10 OSO10

Category: Technical issue with the UAS

Topic: Safe recovery from a technical issue

#### 3.4.5.10.1 Level of Integrity

- It cannot be expected that when operating overpopulated areas or assemblies of people a fatality will not occur from any probable failure of the UAS or any external system supporting the operation, however:

- the operation does take place overpopulated areas but not over assemblies of people, and As shown in Section 3, even under the event of crash the chances of single fatalities are low and the risk is acceptable.

- All pilots are trained to select ELZ on all missions at the Flight Planning stage, they will also Identify them when on site, & continuously update them as the Drone moves through its flight mission.

- All Pilots are trained on Emergency procedures and have a course completion certificate   to state this. This training is part of the Internal Flight Training course.

Based on the above:

Level of integrity OSO09: LOW/MEDIUM/HIGH

#### 3.4.5.10.1 Level of assurance

- The low integrity requirement is not satisfied.

- Risks to the CONOPs are considered: e.g., weather limitations.

Level of assurance OSO09: LOW/MEDIUM/HIGH

#### 6.1.5.10.3 Current Level of Robustness

Based on the above:

Level of Robustness OSO09: LOW/MEDIUM/HIGH

#### 3.4.5.11. OSO11

Category: Deterioration of external systems supporting UAS operations

Topic: Procedures are in-place to handle the deterioration of external systems supporting UAS operations

Covered with OSO08.

#### 3.4.5.12 OSO12

Category: Deterioration of external systems supporting UAS operations

Topic: The UAS is designed to manage the deterioration of external systems supporting UAS operations

Covered with OSO10.

#### 3.4.5.13 OSO13

Category: Deterioration of external systems supporting UAS operations

Topic: External services supporting UAS operations are adequate for the operation

#### 3.4.5.9.13.1 Level of Integrity

Your Company Name ensures that the level of performance for any externally provided service necessary for the safety of the flight is adequate for the intended operation.

Based on the above:

Level of integrity OSO13: LOW/MEDIUM/HIGH

#### 3.4.5.13.2 Level of assurance

Your Company Name declares that the requested level of performance for any externally provided service necessary for the safety of the flight is achieved (without evidence being necessarily available)

Based on the above:

Level of assurance OSO13: LOW/MEDIUM/HIGH

#### 3.4.5.13.3 Current Level of Robustness

Combining both the level of assurance and integrity:

Level of robustness OSO13: LOW/MEDIUM/HIGH

#### 3.4.5.14 OSO14

Category: Human Error

Topic: Operational procedures are defined, validated, and adhered to

Covered with OSO08.

#### 3.4.5.15 OSO15

Category: Human Error

Topic: Remote crew trained and current and able to control the abnormal situation

Covered with OSO09

#### 3.4.5.16 OSO16

Category: Human Error

Topic: Multi-crew coordination

#### 3.4.5.16.1 Level of Integrity

Based on the levels of integrity of the different requirements explained below:

Level of integrity OSO16: LOW/MEDIUM/HIGH

Requirement 1

Procedures to ensure coordination between the crew members and robust and effective communication channels are available and at a minimum cover:

- assignment of tasks to the crew and (b) establishment of step-by-step communications.

Level of integrity requirement 1: LOW/MEDIUM/HIGH

Requirement 2

The crew training covers the coordination between different crew members in terms of communications, roles, and responsibilities, see Section 3.7.

Level of integrity requirement 2: LOW/MEDIUM/HIGH

Requirement 3

N/A

Level of integrity requirement 3: LOW/MEDIUM/HIGH

#### 3.4.5.16.2 Level of assurance

The overall level assurance for this OSO based on the Requirements exposed below is:

Level of assurance OSO16: LOW/MEDIUM/HIGH

Requirement 1

- The procedures are defined according to the EASA standards (EU) 2019/947.

- The adequacy of the procedures has been proven though out more than 1000 VLOS flight hours.

Level of assurance requirement 1: LOW/MEDIUM/HIGH

Requirement 2

- The training syllabus is available in Section 3.7.

- A DUTO Recognised Entity provides this training competency-based, theoretical, and practical training.

Level of assurance requirement 2: LOW/MEDIUM/HIGH

Requirement 3

N/A

#### 3.4.5.16.3 Level of Robustness

Considering the levels of assurance and integrity for this OSO, the current level of robustness is:

Level of robustness OSO16: LOW/MEDIUM/HIGH

#### 3.4.5.17 OSO17

Category: Human Error

Topic: Remote crew is fit to operate

#### 3.4.5.17.1 Level of Integrity

Your Company Name has a policy defining how the crew can declare themselves fit to operate before conducting any operation, see 3.4.2 and 3.4.3 in OM.

- The duty and resting times are defined an adequate for the operation.

- The fitness of the crew members (IMSAFE) is checked during the flight preparations. Based on the above, the level of integrity is:

Level of integrity OSO17: LOW/MEDIUM/HIGH

#### 3.4.5.17.2  Level of assurance

- The crew duty, flight duty and the resting times policy are documented.

- The crew duty cycles are logged and cover at a minimum.

However,

- no evidence is registered that the crew is fit to operate.

Based on the above, the level of integrity is:

Level of assurance OSO17: LOW/MEDIUM/HIGH

#### 3.4.5.17.3 Level of Robustness

Considering the levels of assurance and integrity for this OSO, the current level of robustness is:

Level of robustness OSO17: LOW/MEDIUM/HIGH

#### 3.4.5.18 OSO18

Category: Human Error

Topic: Automatic protection of the flight envelope from human error

#### 3.4.5.18.1 Level of Integrity

Sensefly & DJI Flight control system incorporates automatic protection of the flight envelope to prevent the remote pilot from making any single input under normal operating conditions that would cause the UAS to exceed its flight envelope or prevent it from recovering in a timely fashion.

Based on the above, the level of integrity is:

Level of integrity OSO1: LOW/MEDIUM/HIGH

#### 3.4.5.18.2 Level of assurance

The automatic protection of the flight envelope has been developed in-house, without following specific standards.

Based on the above, the level of integrity is:

Level of assurance OSO18: LOW/MEDIUM/HIGH

#### 3.4.5.18.3 Level of Robustness

Considering the levels of assurance and integrity for this OSO, the current level of robustness is:

Level of robustness OSO18: LOW/MEDIUM/HIGH

#### 3.4.5.19 OSO19

Category: Human Error

Topic: Safe recovery from human error

#### 3.4.5.19.1 Level of Integrity

Based on the combination of the different requirements stated below the level of integrity is:

Level of integrity OSO 19: LOW/MEDIUM/HIGH

Requirement 1 Procedures and Checklists

Procedures and checklists that mitigate the risk of potential human errors from any person involved with the mission are defined.

Level of integrity requirement 1: LOW/MEDIUM/HIGH

Requirement 2 Training

- The crew is trained to use checklists and procedures, however

- the training does not include a specific CRM currently but is design to ensure safe and efficient operations.

Level of integrity requirement 2: LOW/MEDIUM/HIGH

Requirement 3 UAS Design

SenseFly & DJI systems detecting and/or recovering from human errors are developed according to industry best practices.

Level of integrity requirement 3: LOW/MEDIUM/HIGH

#### 3.4.5.19.2 Level of assurance

Based on the combination of the different requirements the level of assurance of this OSO is:

Level of assurance OSO 19: LOW/MEDIUM/HIGH

Requirement 1 Procedures and Checklists

- Procedures and checklists are validated against a standard or a means of compliance considered adequate by the competent authority.

- The adequacy of the procedures and checklists is based on Your Company Name experience acquired during more than 1000 VLOS flight hours.

- Dedicated flight test is put in place to validate the procedures.

Based on the above, the level of assurance of this requirement is:

Level of assurance requirement 1: LOW/MEDIUM/HIGH

Requirement 2 Training

This level of assurance is based on the level of assurance of OSOs 9, 15 and 22.

Based on the above, the level of assurance of this requirement is:

Level of assurance requirement 2: LOW/MEDIUM/HIGH

Requirement 3 UAS Design

Your Company Name declares that the required level of integrity has been acquired. Your Company Name do not design Drones. All Your Company Name drones are from a UAS manufacturer

Based on the above, the level of assurance of this requirement is:

Level of assurance requirement 3: LOW/MEDIUM/HIGH

#### 3.4.5.19.3 Level of Robustness

Based on the above, the level of assurance of this OSO is:

Level of robustness OSO 19: LOW/MEDIUM/HIGH

#### 3.4.5.20 OSO20

Category: Human Error

Topic: A human factors evaluation has been performed and the human machine interface (HMI) found appropriate for the mission

#### 3.4.5.20.1 Level of Integrity

The UAS information and control interfaces are clearly and succinctly presented and do not confuse, cause unreasonable fatigue, or contribute to remote crew errors that could adversely affect the safety of the operation.

Based on the above:

Level of integrity OSO 20: LOW/MEDIUM/HIGH

#### 3.4.5.20.2 Level of assurance

Your Company Name has designed the procedures and training to improve the HMI experience of the crew, increasing the situation awareness of the crew and improving the aeronautical decision making.

Based on the above:

Level of assurance OSO 20: LOW/MEDIUM/HIGH

#### 3.4.5.20.3 Level of Robustness

Based on the above:

Level of robustness OSO 20: LOW/MEDIUM/HIGH

#### 3.4.5.21 OSO21

Category: Adverse operating conditions

Topic: Operational procedures are defined, validated, and adhered to

Covered with OSO08.

#### 3.4.5.22 OSO22

Category: Adverse operating conditions

Topic: The remote crew is trained to identify critical environmental conditions and to avoid them

Covered with OSO09

#### 3.4.5.23 OSO23

Category: Adverse operating conditions

Topic: Environmental conditions for safe operations are defined, measurable and adhered to

#### 3.4.5.23.1 Level of Integrity

The level of integrity of this OSO is defined by the level of assurance:

Level of integrity OSO 23: Level of assurance OSO 23: LOW/MEDIUM/HIGH

Requirement 1 Definition

The environmental conditions for safe operations are defined in this operations manual

Level of integrity requirement 1: Level of assurance requirement 1: LOW/MEDIUM/HIGH

Requirement 2 Procedures

Procedures to evaluate environmental conditions before and during the mission (i.e., real-time evaluation) are available and include assessment of meteorological conditions with a simple recording system.

Level of integrity requirement 2: Level of assurance requirement 2: LOW/MEDIUM/HIGH

Requirement 1 Training

The training covers the assessment of meteorological conditions.

Level of integrity requirement 3: Level of assurance requirement 3 : LOW/MEDIUM/HIGH

#### 3.4.5.23.2 Level of assurance

Based on the analysis of the different Requirements the resultant level of assurance is:

Level of assurance OSO 23: LOW/MEDIUM/HIGH

Requirement 1 Definition

Your Company Name can provide support of this evidence beyond the information stated in this operations manual.

Level of assurance requirement 1: LOW/MEDIUM/HIGH

Requirement 2 Procedures

- The procedures are validated by standard means of compliance of the competent authority.

- Dedicated flight test is put in place to validate the procedures.

Level of assurance requirement 2: LOW/MEDIUM/HIGH

Requirement 1 Training

- The training syllabus is available in Section 3.7.

- DUTO Provides competency-based, theoretical, and practical training.

Level of assurance requirement 3: LOW/MEDIUM/HIGH

#### 3.4.5.23.3 Level of Robustness

Based on the above the level of robustness of this OSO is:

Level of robustness OSO 23: LOW/MEDIUM/HIGH

#### 3.4.5.24 OSO24

Category: Adverse operating conditions

Topic: UAS is designed and qualified for adverse environmental conditions

#### 3.4.5.24.1 Level of Integrity

The UAS designed to limit the effect of environmental conditions stated in Section 3.

Level of integrity OSO 24: LOW/MEDIUM/HIGH

#### 3.4.5.24.2 Level of Robustness

Your Company Name declares that the level of integrity has been achieved based on 1000h of VLOS flight hours and the level of experience the Remote Pilots have.

Level of assurance OSO 24: LOW/MEDIUM/HIGH

#### 3.4.5.25 OSO25

Category: Construction sites

Topic: Operational procedures are defined, validated and adhered to

Covered with OSO08.

#### 3.4.5.26 OSO26

Category: Construction sites

Topic: The Remote Pilot is trained to identify critical environmental conditions or changes and to avoid them

Covered with OSO09.

#### 3.4.5.27 OSO27

Category: Construction sites

Topic:  Environmental conditions for safe operations are defined, measurable and adhered to

Covered with OSO23

#### 3.4.5.28 OSO28

Category: Construction sites

Topic:  Flight Geography and ground risk adhered to

#### 3.4.5.28.1 Level of Integrity

The Flight Geography area is the Construction Site whole. This is a controlled environment with safety heavily adhered to. The Drone has a geofence capability to keep the drone in a boundary box. The Distance and height can be inputted into the Flight app and the Drone will not leave this boundary box, once GPS is functioning. See Section 3.1.3.4. All Involved/Uninvolved person wear Personal Protective equipment. Hard Hats, Hi-vis vest, long sleeves gloves and Eye goggles. On terminating the flight the drone will not cause injury to persons because of the PPE on all Persons.

Level of integrity OSO 28: LOW/MEDIUM/HIGH

#### 3.4.5.28.2 Level of assurance

All Personal must attend a induction prior to entering a construction site this outlines what they can and cant do. If they fail to follow these rules they will be asked to leave site. There is a record of their induction. If a hard hat is removed from a persons while on site they will be asked to leave site. It is a policed environment therefore the injury to persons is greatly reduced.

Level of assurance OSO 28: LOW/MEDIUM/HIGH

#### 3.4.5.28.2 Level of Robustness

The Level of Robustness is medium

Level of assurance OSO 28: LOW/MEDIUM/HIGH

#### 3.4.5.29 OSO29

Category: Construction sites

Topic:  Flight Geography and Air Risk adhered to

#### 3.4.5.29.1 Level of Integrity

The Flight Geography area is the Construction Site whole. This is a controlled environment with safety heavily adhered to. The Drone has a geofence capability to keep the drone in a boundary box. The Distance and height can be inputted into the Flight app and the Drone will not leave this boundary box, once GPS is functioning. See Section 3.1.3.4. This Boundary box allows maximum flight height to be inputted. In controlled airspace when operating within 5km of an airport ATC will control RP’s flight height, this shall be inputted into Flight App.

Level of integrity OSO 29: LOW/MEDIUM/HIGH

#### 3.4.5.29.2 Level of assurance

The RP has been trained in how to set up the Boundary Box prior to any flight mission. U.F 101 applications need to submit as part of Your Company Name  Operating procedures.

Level of assurance OSO 29: LOW/MEDIUM/HIGH

#### 3.4.5.29.2 Level of Robustness

The Level of Robustness is medium

Level of assurance OSO 29: LOW/MEDIUM/HIGH

#### 3.4.5.30 OSO30

Category: Quarry sites

Topic:  Flight Geography and Ground Risk adhered to

Covered with OSO28

#### 3.4.5.31 OSO31

Category: Quarry sites

Topic:  Flight Geography and Air Risk adhered to

Covered with OSO29

#### 3.4.5.32 OSO32

Category: Quarry sites

Topic:  The Remote Pilot is trained to identify critical changes to environmental conditions, Dust Clouds, Blast areas

#### 3.4.5.32.1 Level of Integrity

Prior to RP conducting Operations in Quarry Sites they must attend a Induction for the site. During this induction the RP is informed of Blast times and how long the dust cloud will settle. The RP must follow these rules. As covered with OSO 29 the Ground Risk injury to persons is reduced in the event of the flight needs to be terminated. All RPs are trained in how to operate in Quarries. This training would be part of the RP live on the job training with a full qualified RP. The RP is trained to locate themselves in a position that has full view of the Quarry Site

Level of integrity OSO 32: LOW/MEDIUM/HIGH

#### 3.4.5.32.2 Level of assurance

The RP Flight training is recorded in the Company personal file. This is is reviewed every 6 months when internal training happens for all RP’s

Level of assurance OSO 32: LOW/MEDIUM/HIGH

#### 3.4.5.32.2 Level of Robustness

The Level of Robustness is medium

Level of assurance OSO 32: LOW/MEDIUM/HIGH

## 3.5 Technical means used.

These sections will outline main characteristics, performance, and limitations, including UAS, external systems supporting the UAS operation, facilities

### 3.5.1 Multi-Rotor DJI Mavic 2 Enterprise Advanced

| DJI Mavic 2 Enterprise Advanced |  |
| --- | --- |
| Image 27:Mavic 2 Enterprise Advanced |  |
| The Aircraft |  |
| The composite of the aircraft is a mix of Plastic and Carbon Fibre. It is a durable and robust airframe. It has a diagonal span of 350mm. The Maximum take-off weight is 1100g. It has the ability to fly at slow speeds Manually and in Semi-Autonomous mode, e.g., Flight plan mapping missions. It has the ability to terminate flight at any given time by the RP. |  |
| The Main Characters of the Mavic 2 Enterprise Advanced |  |
| Weight | 909grams – MTOW 1100G |
| Diagonal Wheelbase | 354mm |
| Max Ascent Speed | P-mode: 5m/s S – Mode: 6m/s |
| Max flight Speed | P-mode: 14m/s S-mode: 20m/s |
| Max Wind Speed Resistance | 10m/s |
| Max Service Ceiling Above Sea Level | 19685 feet (6000 m) |
| Max Flight Time | Approx. 28mins |
| Operating Temperature Range | 20° to 50°C |
| Ingress Protection Rating | NO IP rating |
| Satellite Positioning Systems | GPS+GLONASS+BeiDou+Galileo |
| Infrared Sensing System |  |
| Infrared Sensing System Distance | 0.6 – 23feet (0.2 – 7meters) |
| Operating Environment | reflectivity >8% |
| Camera |  |
| RGB Sensor | 1/2” CMOS, Effective Pixels: 48 M |
| Gimbal | 3 Axis Stabilisation system |
| Thermal Sensors | Uncooled VOx Microbolometer |
| Remote Controller |  |
| Max Transmission Distance | 7km - 2.4 GHz - 5.8GHz |
| Transmitter Power (EIRP) | 2.4 GHz FHSS link (106 to 63 mW) |
| Operating Current/Voltage | 3850 mAh weight 297g |
| See Manufacturers User Manual for further information. This User Manual is part of our additional documentation and we have it recorded as such in our list of references table. Please see section 1.2.2 List of Refence Documents |  |

## 3.6 Competency, Duties and Responsibilities

| Role | Name | Contact |  |
| --- | --- | --- | --- |
| Head of Drone Operations |  |  |  |
| Safety Manager |  |  |  |
| Head of Training | External Company |  |  |
| Maintenance Manger |  |  |  |
| Remote Pilot/Chief |  |  |  |
| Remote Pilot |  |  |  |
| Visual Observer (VO) |  |  |  |
| Remote Pilot |  |  |  |
| Remote Pilot |  |  |  |

Table 11: List of Pilots

### 3.6.1 Head of Drone Operations.

The Head of Drone Operations is a highly responsible role. Which will be included and be not limited to the following. They must be trained under the current Drone regulations for Drone Operations within the Land of Ireland. They may not hold a Drone Licence, if they wish not to, but have the relevant courses completed. This will ensure they fully understand what is required from holding this role. They can apply for a Drone Licence if they wish to do so.

The qualifications required for this role are as follows.

- Must be Registered within the MySRS Portal as a key person under “Responsible Manager” for Your Company Name.

- A1/A3 Proof of online training completed on MySRS Portal.

- A2 Remote Pilot Competency Certificate

- STS-01/PDRA-01 under the Specific Category

The role and responsibilities for this appointment within Your Company Name.

- Responsible authority who must ensure all internal procedures are followed in line with process descriptions, laws, and safety management system.

- Ensuring all departments have enough resources to perform their works.

- Focal point to the management board.

- Responsible for Ireland & UK flight operations.

- Coordination and communication with training organisation, maintenance organisation and project management.

- Ensuring personnel is trained and qualified for intended operations.

- Design and maintenance of Operations Manuals and CONOPS.

- Elaboration and approval of risk assessments.

- Coordination of Ireland & UK operations.

- Focal point for Flight Crew

- Design and Validation of flight routes and operational procedures.

- Communications with the department of civil aviation: submission of documentation and procurement of approvals.

### 3.6.2 Safety Manager

The Role of Safety Manager is one of importance, they must have a health and safety background. They must understand the current Drone Rules and Regulations with regards to the Air and Ground Risk that is involved in Drone Operations. They may not need to undergo any external Drone Courses, as the Head of training, can explain what is required from the Air and Ground Risk for Drone Operations. The Head of Training can engage with the Safety Manager on the drafting up of any new Risk based documentation to ensure the correct approach is given for Drone Operations. The Safety Manager will oversee all Health and Safety aspects of any Drone operations within Ireland and the UK for Your Company Name.

The qualifications required for this role are as follows

- Must be registered with the IAA MySRS Portal as a Key Person “UAS – Safety Manager” for Your Company Name.

- A1/A3 Proof of online training completed on MySRS Portal.

- Received some training from the Head of training regarding Air and Ground Risk under the current Rules and Regulations for Drone Operations within Ireland & the UK.

The role and responsibilities for this appointment within Your Company Name.

- Facilitate hazard identification, risk analysis and risk management.

- Monitor implementation of risk mitigations and provide periodic reports.

- Designing and maintaining safety management system including organisational handbooks and ensuring safety management trainings.

- Occurrence investigation initiation and follow-up.

- Coordination and communication with authorities regarding incident/accident investigations.

### 3.6.3 Head of Training

This Role can be held by the same person who is also the Chief Remote Pilot. The type of training they can conduct will be company workflows and flight training on company aircrafts. They cannot certify anyone for Drone Ground School or conduct any Flight Exams. This can only be performed by an IAA Recognised Entity. This person should touch base with the Recognised Entity at least twice a year asking for any changes on Drone Rules and Regulations.

The qualifications required for this role are as follows.

- Must be Registered within the MySRS Portal as a key person under “UAS – Responsible Person/Nominated Person – Flight Operations” for Your Company Name.

- A1/A3 Proof of online training completed on MySRS Portal.

- A2 Remote Pilot Competency Certificate

- STS-01/PDRA-01 VLOS Operations under the Specific Category

- STS-02/PDRA-02 BVLOS Operations under the Specific Category (Optional)

The role and responsibilities for this appointment within Your Company Name.

- Design and maintenance of training standards.

- Organisation of trainings for flight crew and maintenance crew.

- Organisation and qualifications of internal and external trainers and partners.

- Performing trainings (basic, advanced, BVLOS…).

- Design and maintenance of training organisation handbook.

- Monitoring compliance with terms of approval / privileges, risk assessment and mitigations, manuals and procedures, training standards.

- Perform internal audits.

- Consulting operational, trainings, maintenance, and safety department with regards to regulations and regulatory requirements.

- Monitoring compliance with applicable regulations (operational, technical…)

### 3.6.4 Maintenance Manger

This Role can be held by the same person who is also the Chief Remote Pilot. The Maintenance Manger must know and understand all features of the company’s Drones. They must have read all Manufacturers Manuals/Instructions on the Company’s Drones. They must keep a record of any incidents of any changes to the Aircraft. This includes Firmware Versions and Software Versions. They will liaise with the Drone Manufacturers or Country Agents if a Drone must need to be shipped back for repair. They will be the first point of contact in the unlikely event of an incident. In the event of a test flight after repair the Maintenance Manager can either conduct it themselves once they have the minimum of holding an A1/A3 Tab on their Drone Licence or they request the Chief remote pilot to complete the test flight for them under their watchful eye.

The qualifications required for this role are as follows.

- Must be Registered within the MySRS Portal as a key person under “Maintenance Manager” for Your Company Name.

- A1/A3 Proof of online training completed on MySRS Portal.

The role and responsibilities for this appointment within Your Company Name.

- Ensuring all maintenance requirements developed by Manufacturers are followed and coordination with Manufacturers or Country agent for the shipment of Drones in the event of maintenance/repair.

- Ensuring all maintenance tasks are performed on all Companies Drones if support is requested and all maintenance / repair logs are filled.

- Ensuring maintenance personnel is qualified and trained.

- Design and maintenance of maintenance organisation handbook/tables.

- Communication and coordination with internal and external stakeholders with regards to all maintenance aspects.

### 3.6.5 Remote Pilot/Chief

The Remote Pilot or Chief Remote Pilot have the final say on whether the Drone operation will take place or not, they are entitled to cancel an operation if he/she is not satisfied with the prevailing conditions. The Head of Operations will task and launch the RP/CRP to conduct flight planning for all Drone Operations. The Chief Remote pilot will then task each Remote Pilot to a project. The Remote Pilot will report back once this is completed and await further taskings from the CRP.

The qualifications required for this role are as follows.

- Must be Registered within the MySRS Portal as a key person under “UAS Responsible Person” for Your Company Name.

- A1/A3 Proof of online training completed on MySRS Portal.

- A2 Remote Pilot Competency Certificate

- STS-01/PDRA-01 under the Specific Category

- STS-02/PDRA-02 BVLOS Operations under the Specific Category (Optional)

The role and responsibilities for this appointment within Your Company Name.

The Chief Remote Pilot is highest authority on site and is responsible for the execution OR supervision of the implementation of the procedures defined in this OM.

Flight Preparations

- Ensure that all the necessary permissions are current and present.

- Ensure that all the crew members are qualified for their role and type of operation.

- Ensure that the third-party insurance of the UAS is current, present and covers all the residual risks of the operation.

- Flight plan filing.

Pre-Flight

- Ensure that the crew including oneself are fit to fly (IMSAFE).

- Ensure that the operating environment (PAVE) satisfies the declared and authorized operational conditions.

- Conduct a safety briefing/project briefing.

- Ensure that the UAS is fit to fly and complies with the instructions and limitations provided by the manufacturer.

- Ensure that the Remote Controller is properly set up and the emergency equipment is accessible.

- Ensure that any payload carried is properly secured and installed, respecting the limits for the mass and CG of the UAS.

- Ensure that the UAS has enough propulsion energy for the intended operation based on:

(i) the planned operation; and

(ii) the need for extra energy in case of unpredictable events

- Verify that the weather conditions at the time of the start as well as those expected for the entire period of the operation are compatible with those defined in the AOM, OM or CONOPs whatever is more restrictive or applicable.

- Plan of flight missions in QGC according to this OM and the UAS limitations described in the AOM, whatever is more restrictive.

- Validate the BVLOS missions together with the FOM.

- Get familiar with the environment and obstacles through scouting flights and survey.

Flight Execution

- Supervise the operation and coordinate the roles of the other crew members.

- Avoid any risk of collision with any manned aircraft and abort a flight when continuing it may pose a risk to other aircraft, people, animals, environment, or property.

Post-Fight

- Documentation of flight data.

- Debriefing

The Chief Remote Pilot can carry out maximum his role as well as the role of the RP simultaneously.

### 3.6.6 Remote Pilot

The Remote Pilot is the drone pilot for a project. The RP can work alone, or under the supervision of the CRP. He/she will be ensuring all relevant paperwork is completed such as pilot & UAS hours, battery logs etc. He/she will be ensuring all charging and checks prior to deployment are made this includes.

- Maintenance checks are carried out on the systems

- All batteries are charged and checked

- All ancillaries are checked

The RP is responsible for the following:

Pre-Flight

- Assembly and inspection of the UAS.

- Execution of the hover test.

- Setting up the control unit and disposition of the emergency equipment.

- Uploading the correct mission and flight parameters.

- Initiation of the mission.

- Communications with the VO if present/required.

- Payload handling.

- Charging batteries

Flight Execution

- Monitoring the flight parameters.

- Monitoring of weather conditions.

- Monitoring of air traffic.

- Execution of abnormal and emergency procedures.

- Termination of the mission.

- Communications with the VO if Present/required.

Post-Flight

- Disassembly of the UAS

- Inspection for new damages

- Downloading of log files/Data

### 3.6.7 Visual Observer (VO - Optional)

The VO is normally an external person that takes part in the operation on the extreme end of VLOS. The VO is trained by the Head of Training or the CRP. They do not need to be a licensed Drone Pilot. But having completed the A1/A3 Open Category will give them a bit of understanding of the Rules and Regulations under the Open Category and what they should be looking out for. As one of the Air Risk factors would be from other Drone users and there is a big portion that operate under the Open Category. There is no role in the IAA MySRS portal for this position. It is one that the company may decide to implement if and when the need to do so should arise. The Training of such a person will be internal as there is no official VO course available yet. A VO is a must for BVLOS Operations.

The qualifications required for this role are as follows.

- A1/A3 Proof of online training completed on MySRS Portal. (Optional)

Once the VO is validated, he/she carries on the following task:

- Clearing the landing area of obstacles, animals, and people.

- Provides the RP with information relative to the UAS.

- Ensures that any payload carried is properly secured and installed.

### 3.6.8 Competency - Fitness for Duty

All involved in the operation of UASs are required to consider their fitness for duty prior to their participation. The Remote Pilot is entitled to deny participation in the operation to anyone if the Remote Pilot considers he/she is not fit.

Prior to the operation all crew members must go through the following check “IMSAFE”:

- Illness

- Medication

- Stress

- Alcohol

- Fatigue

- Emotional load

If any of the previous may have an impact in the safety of the operation, the Remote Pilot must: Substitute the crew member, OR Assign him to a less relevant role, OR

Cancel the operation. If the Remote Pilot is looking to Cancel an Operation, they first must contact the Chief Remote Pilot and he does not respond the RP can then directly call the Head of the UAS Department to request permission to Cancel Drone Operations.

### 3.6.9 Competency - Shift & Working Hours

The operation is split into shifts to structure the daily activities and establish routines. The shifts are divided into:

- Flight preparations: activities involving permission requests, flight plan elaboration, mission planning, equipment review…

- Pre-Flight: activities involving assembly and checks of the UAS and CU, as well as fitness of crew members. Weather and prevailing conditions review and briefing.

- Flight Execution: including take-off, cruise, delivery, and landing.

- Post-Flight: activities including debriefing, documentation, and maintenance.

Regarding the allowed working hours, duration of shifts and rest time:

- The maximum number of working hours with flight duty per day is 8 h.

- These hours can be interrupted / paused when the pause is in minimum 90 minutes and the crew member can rest adequately.

- The crew member must rest a minimum of 13 h between duty times.

- Only once (1) in a period of 7 days and if approved by the FOM (Flight Operations Manager) a crew member is permitted to extend the duty time to a maximum of 14h. In that case, the crew rest to be followed is in minimum the duration of the duty time ending.

The duty time starts 60 min before the UAS first take-off and ends 30 min after the UAS last landing of the day. This time is broken up with time being on Duty and Flight hours. The Maximum Flight hours one RP can complete in one day will be 5 hours continuous flying. If the RP feels at any stage during the duty time to rest, they must inform the CRP as a replacement maybe required if RP is operating alone.

### 3.6.10 Competency - Medication & Wellbeing.

At no stage will a RP be under the influence or part take in the consumption of alcohol, narcotics and drugs, sleep aids and anti-depressants these will impair the decision-making during Drone Operations. A RP must declare prior to taking up Duty weather they have taking any of the above substances.

Medication and vaccinations will be allowed once prescribed by a medical doctor, Your Company Name have the right to request a doctor’s note stating this RP is fit for Drone operations under the prescribed Medication & Vaccination.

Fatigue, Stress and Pressure are real when it comes with Drone operations. If at any stage the RP feels any of the above, they must stop Drone Operations and contact the CRP or the Head of the UAS Department. A RP must not fly a Company Drone while they feel Fatigued, Stressed or Pressured. The RP has the final say whether a Drone Project will take place under their command. They must inform the CRP or the Head of the UAS Department if they are looking to Cancel a Drone Project.

## 3.7 Training

Training will be broken up into Phases. Each phase the student must complete before moving on to the next phase. It will be up to the Head of the UAS Department to make sure the RP is fully qualified before they can start any Drone Operations.

Phase One.

Registration on the IAA MySRS Portal.

Complete online A1/A3 Open Category Course.

Start Application process for the A2 Open Category Course.

Theoretical Ground School and Written Exam is conducted through a DUTO.

Complete the A2 Open Category Application

Phase Two.

Internal or External Flight Training on Multi-Rotor.

Flight Exam conducted by DUTO.

Apply for PDRA-01/STS-01 for Operational Authorisation.

Phase Three.

Flight Operations under supervision of experienced RP.

Internal Upskill and recurrence training – 6 monthly.

3.8 Risk analysis and methods for reduction of identified risks

The control volume of the UAS during the operation is shown Figure 5. The information is extracted from the EASA SORA analysis. The dimension of the different volumes and buffers is as follows:

- Contingency volume: 300m left and right of the flight path.

- Emergency volume: 300m left and right of the flight path.

- Ground risk buffer: as a general approach follow the 1:1 rule, unless specified otherwise in the risk assessment.

- Air risk buffer: as a general approach follow the 1:1 rule, unless specified otherwise in the risk assessment

Image 28:Ground Risk / Air Risk Model

## 3.9 Maintenance

The maintenance scheduled or unscheduled and the repairs are coordinated by the maintenance manager. On site, maintenance is performed by the Maintenance Technician according to the checks defined in Manufacturer’s Instructions Manual. See 1.2.2 List of Refence Documents  Every scheduled or unscheduled maintenance work or repair is register in the maintenance log. All internal Maintenance of the Aircrafts will be conducted by the Manufacturers of the Aircraft.

# 4. Normal procedures.

The standard operation procedures are described using the different flight phases as structure: preparation phase, pre-flight, flight execution and post-flight

## 4.1 General procedures valid for all operations:

Multi-Rotor Aircraft.

Setup Procedures

- Open Carry Case.

- Take out Drone.

- Visual Inspection of aircraft body for damage.

- Place Drone on the ground.

- Take out Flight Battery.

- Visually inspect this for damage.

- Insert into drone.

- Take out remote controller.

- Visually inspect for damage.

Start-up procedures

- Switch on RC and wait for Software to boot up.

- Switch on Drone and wait for Self-Check conformation tone.

- Activate any unlocking codes if required for site (DJI)

- Confirm Home Position is recorded and correct check it on the Map. (DJI)

- Stand Back and Observe Take-off Area.

- Open flight mission on Table/Phone/Built-in Screen/Computer.

- Check parameters – Edit if need be.

- Upload mission to Aircraft.

- Check Take-off Area for any new hazards.

- Prior to take-off observe Zero Meters in Altitude (+/- 0.9m).

- Start Mission.

- Observe Present Telemetry on Table/Phone/Built-in Screen/Computer.

- Observe Drone at take-off for approx.10meters in Altitude then observe between Telemetry and Drone until the Drone is at the Start of the Mission Flight Start waypoint.

Inflight procedures

- Monitor Drone in Flight while in Semi-Autonomous flight mission.

- Check progress of Mission Plan toggle between Camera live screen and Map screen on App.

- Observe General Flight Area outside of Flight Mission Area for other air users.

- No Phone Calls will be taken by RP during Semi-Autonomous flight missions - The Exception is only when working inside Controlled Airspace and ATC may be ringing RP to give instructions.

- Be prepared to Cancel Flight Mission at any time by the use of RTH Function in the unlikely event of an Emergency Procedure.

Point to Note: RTH Function is only to be used as an Emergency function and not as a standard method to return the aircraft to your position.

Landing procedures

- On termination or completion of a Semi-Autonomous flight mission the Drone will go into RTH – RP will observe this happening he will let the Drone complete this function if he is happy with the Return Flight Path the Drone is taking. If not, the RP will take over manual control of the Drone and land it.

- The RP will observe the Drone During the RTH process. If they need to take Manual control at any stage they will.

- The RP will make sure the landing site is clear from any hazards.

- The RP will monitor the flight telemetry on the return home.

- Prior to the drone landing at the home position the RP will be ready to take Manual Control of the Drone.

- Once landed RP will walk over a switch off Drone.

Post-flight procedures

- Visual Inspection of aircraft body for Damage.

- Visual Inspection of Propellers for damage.

- Fill in Logbooks.

- Debrief the Mission.

## 4.2 Procedures peculiar to a single operation

See section 4.1 General procedures valid for all operations with the following functions.

Setup Procedures (Controlled Airspace)

- Once aircraft is assembled and ready to fly the RP with Contact ATC to get permission to take-off.

- Contact second ATC if working in Dublin CTR (Weston/Baldonnell Airspace).

Inflight procedures (Controlled Airspace)

- Mobile Phone of RP is only used to answer Phone calls from ATC all other phone calls will be ignored until Drone Flight mission is complete, and Drone has landed.

- Respond to any request from ATC.

Landing procedures (Controlled Airspace)

- Observe airspace for other air users.

- Respond to any request from ATC.

Post-flight procedures (Controlled Airspace)

Ring ATC to inform them that you are done and are finished on current operations.

# 5. Contingency procedures

Contingency Procedures are written up in the unlikely event of a possible change in Drone operations for whatever reason.

## 5.1 General procedures valid for all operations

Image 29:Operational Volume

The contingency procedures are those applied under abnormal situations which would not compromise the controllability and stability of the UAS posing an imminent danger to those on the ground or in the air. The Operational Volume of your Operations allow for Contingencies to take place once the Drone Leaves the Flight Geography Area. This area gives the pilot extra time to either return the drone to the Flight Geography Area or land the Drone in the Contingency area of your Flight Operational Volume.

### 5.1.1 Sudden Change in Weather into adverse conditions Actions.

This situation occurs when the environmental conditions in the area of operations deteriorate into adverse operating conditions

| Remote Pilot | Crew Actions (if present) |
| --- | --- |
| 1. The Remote Pilot must identify the deterioration. 2. If the deterioration is reduced visibility and it looks visual contact with UAS might be lost, then UAS operations should stop immediately. 3. If the deterioration is multiplied wind speed on the far side UAS flight envelope or what's thought-about safe by RC for the present operations, then UAS operations should cease immediately. 4. If the deterioration may be a reduced air temperature to the purpose of attainable craft icing, then UAS operations should cease immediately. | 1. RTH Function can be activated. 2. Go to Force Landing Procedure |

### 5.1.2 UAS Departs the Flight Geography Actions

This situation occurs when the UAS departs the Flight Geography and enters the Contingency Volume/Containment Volume.

The UAS may enter the designated Contingency volume as a result of some of the following examples:

- Evasive action: an unexpected obstacle or hazard has entered the flight path and the UAS must deviate to avoid it e.g., Birds or other aircraft.

- Interference: Interference to the UAS’ onboard sensors in either manual or automatic mode may cause the UAS to deviate from the expected flight path.

- Adverse weather: Weather conditions such as high winds could force the UAS outside the desired flight geography.

| Remote Pilot | Crew Actions (if present) |
| --- | --- |
| 1. Attempt to manually correct the UAS’ course. 2. If conditions worsen, go to Force Landing Procedure or activate the RTH Function. 3. If there is clear ground under the UAS and it is safe to do so, terminate the flight at least 10 m before the UA reaches the limits of the operational volume. 4. If none of the above work, please proceed to the “Fly Away Aircraft” procedure. | 1. Alert the Remote Pilot about the UAS incursion into the contingency volume. 2. Monitor the area under the UAS in case a Flight Termination Command is required |

### 5.1.3 Sudden Mission Termination because of Compass Failure Actions.

This situation occurs when the internal compass in the aircraft has an error and not functional correctly. This can be caused by magnetic interference from the surrounding area e.g., deposits of metal, electric lines, etc.

- The function of the aircraft can be severely affected especially during automatic flight operations.

- The RTH failsafe may not work if interference is too strong.

- DJI Products with Compass Error or Failure will enter into Atti Mode this is where you maintain Height/Altitude but not position. Expect drifting when this happens, because of Wind. Drone will Drift at the same speed as the wind.

| Remote Pilot | Crew Actions (if present) |
| --- | --- |
| 1. Cancel any automatic flight operation and ensure the UAS is stopped in a manual mode. 2. Check the control functions of the UAS (forward/back, left/right, pitch /yaw). 3. Land as soon as practical. 4. Perform a compass calibration. | - Inform RP of weather changes. - Inform RP of Wind Direction Changes. - Monitor Weather Apps. |

### 5.1.4 Telemetry Loss during Flight Mission Actions

This could be a situation where C2 Signal is weak or intermitting Remote Pilot will get a heads up warning on flight telemetry.

Points to Note:

- If your C2 Link signal is low, any operation that you command with the control app or RC might need to be attempted multiple times or could be delayed due to the weak C2 Link signal.

- If you transition from a low signal warning to loss of communication refer to the Lost C2 Link emergency procedures section of this document.

| Remote Pilot | Crew Actions (If Present) |
| --- | --- |
| 1. Where applicable cycle the mode switch from P mode to A mode and back to P mode. This is to take the Drone out of any Program mode. It gives Positive control back to the Remote Pilot. 2. Check that RC antennas are pointed up and out towards the UAS. Ensure no-one is standing directly in front of antennas. 3. Ensure distance from home reading in the app is within operational range. 4. Use the Return to Home button on the RC to land the UAS.(If need be try regaining positive control before activating RTH.) 5. Retrieve the UAS. 6. Establish probable cause of loss of signal to avoid a similar situation in future. | 1. Visually confirm if the UAS holds position. 2. Keep constant line of sight with the UAS. 3. Monitor for potential obstructions between the UAS and RC that could cause interference. 4. Visually confirm if the UAS has initiated Return to Home. |

### 5.1.5 Interference Avoidance with Manned Aircraft Actions.

This situation occurs when there are visible or audible signs of an unknown third-party airspace user in the Adjacent Airspace with the intent of not changing course.

- This potentially could become a flight incursion.

| Remote Pilot | Crew Actions (If Present) |
| --- | --- |
| 1. RC should identify the probable course of the other airspace operator. 2. If it is likely that the other airspace operator could enter the operational volume, then start the “Unknown airspace user enters Operation Volume” procedure immediately. | 1. Assist RC in monitoring the unknown airspace operator. 2. Once threat passes by safely contact with ATC may be made to confirm their awareness of current UAS operations. |

# 6. Emergency procedures

The UAS operator should define procedures to cope with emergency situations.

- Procedures to avoid or, at least minimise, harm to third parties in the air or on the ground. With regard to the air risk, an avoidance strategy to minimise the collision risk with another airspace user (in particular, an aircraft with people on board).

(b) Procedures for the emergency recovery of the UA (e.g., landing immediately, termination of the flight with FTS or a controlled crash/splash, etc.).

## 6.1 List of Possible Emergency’s

Below is a list of Emergency Procedures for various scenarios which will be adhered to by the Your Company Name UAS Operator. The procedures are as follows.

- Fail Safe

- Force Landing Procedure

- Software Malfunction

- Transmitter Failure/Frequency Interference

- Loss of Propulsion/Motor or Propeller Failure

- Aircraft Battery Failure

- Loss of GPS Signal

- Low Magnetic Field

- Public Encroachment

- Aircraft Incursion

- Fly Away

- Pilot Incapacitation

- Fire Ground Equipment

- Fire in Flight

Any emergency will be recorded in the Aircraft Hours Logbook under the “notes” section.

## 6.2 Fail Safe Procedure

If the remote-control signal is lost for more than 2 seconds, failsafe Return To Home automatically activates.

- Return to Home

- Once activated the aircraft starts to retrace its original flight route home.

- If the remote-control signal is re-established within 60 seconds the aircraft will hover at its present location for 10 seconds and wait for pilot commands.

- The UAS OPERATOR will press the RTH button on the remote controller to cancel RTH and take control.

- If no command is given the aircraft will fly in a straight line to the Home Point

- If the remote-control signal is still lost after 60 seconds the aircraft will stop retracing its original flight route and flies to Home Point in a straight line.

## 6.3 Force Landing Procedure

The following is the procedure to be followed when a forced landing is required.

Actions:

| Remote Pilot | Actions |
| --- | --- |
| - Remote Pilot-------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- | 1. Call “Force Landing Procedure” . 2. Check for Forced-landing site. 3. Heading for Forced-Landing Site 4. Check for obstacles 5. Check Telemetry a). Speed - controlled & Smooth, b). Flight Height – As Low & Safe as Possible 6. Check Satellites Still in ‘GPS Position’ 7. Calculate consumptions & time left for flight. 8. Scan Landing Site. 9. Speed - Controlled & Smooth |

NOTE

- If in Atti Mode watch flight height and confirm direction of the wind.

- The force landing

## 6.4 Software Malfunction

The following is the procedure to be followed when a software malfunction has been detected.

DETECTION

- The application fails to respond (Jammed).

- Software works slowly.

SIGNIFICANCE

- No control over UAS by application.

- No control over camera.

- No telemetry present.

IMMEDIATE ACTIONS

| Remote Pilot | Actions |
| --- | --- |
| - Remote Pilot-------------------- - Remote Pilot-------------------- - Live Feed------------------------ - Flight Mode--------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- | 1. Call “Software Crash” Reboot Device. 2. Check for Positive Rx from Drone Telemetry After reboot. 3. Check to see if Camera live feed is transmitting 4. Take Positive Control of UAS 5. Continue with Mission 6. Return to Home if Software Crashes a second time. |

ACTIONS

| Remote Pilot | Actions |
| --- | --- |
| - Remote Pilot-------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- | 1. Radio connection. 2. Check for Positive Control from Drone. 3. Check to see if Camera live feed is transmitting 4. Check Positive Control of Camera 5. Check for free movement 6. Return to Home if Issue with Camera. Check Equipment after landing for damage |

NOTE

In Case the Above-Mentioned Procedures Do Not Work.

- Fly by Visual.

- Return to Home Position.

- Altitude maintains until over Home Position.

- Speed - slow and steady until over Home Position.

- Power Consumptions – Are not present continuing flying is extremely dangerous, Land ASAP.

## 6.5 Transmitter Failure/Frequency Interference

The following is the procedure to be followed when a transmitter failure or frequency interference has been detected.

DETECTION

- The UAS fails to respond (Jammed).

- UAS starts RTH command without RP activating the RTH procedure

SIGNIFICANCE

- No control over UAS by application or by Sticks.

- UAS in RTH Mode

- Can’t take UAS out of RTH Mode

IMMEDIATE ACTIONS

| Remote Pilot | Actions on if Communications is re-established |
| --- | --- |
| - Remote Pilot-------------------- - Remote Pilot-------------------- - Live Feed------------------------ - Flight Mode--------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- | 1. Call “Fail Safe – No Control” 2. Check Antenna Positions 3. Check Position and Height of UAS 4. Take Positive Control of UAS 5. Continue with Mission 6. Return to Home if Interference happens a second time. |

| Remote Pilot | Actions on if Communications is not re-established |
| --- | --- |
| - Remote Pilot-------------------- - Remote Pilot-------------------- - Live Feed & Telemetry------- - Flight Mode--------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- | 1. Call “RTH – No Control” 2. Check Antenna Positions 3. Check Position and Height of UAS 4. Take Positive Control of UAS if possible 5. Land ASAP/Forced Landing Procedure ASAP 6. Continue in Return to Home Logic if safe to do so. |

NOTE

- The flight team will then investigate the reasons why the failure occurred, log the incident, and report it to the UAS Accountable Manager

## 6.6 Loss of Propulsion/Motor or Propeller Failure

The following is the procedure to be followed when loss of propulsion or motor failure has been detected.

The UAS Operator will conduct the following actions.

DETECTION

- UAS Ground Station reports Motor Failure.

- UAS responses Sluggishly

- UAS Telemetry shows Motor Failure.

SIGNIFICANCE

- Manual control over UAS by Pilot.

- Monitor telemetry for further Information.

IMMEDIATE ACTIONS

| Remote Pilot | Actions |
| --- | --- |
| - Remote Pilot-------------------- - Altitude--------------------------- - Live Feed------------------------ - Flight Mode--------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- | 1. Call “Motor Failure” 2. Check Height of UAS 3. Check Position and Height of UAS if Changes Land ASAP/Forced Landing Procedure 4. GPS Mode 5. DO NOT Continue with Mission 6. Return to Home to go to Forced Landing Procedure. |

ACTIONS

- UAS-----------------------------------Positive Control.

- Pilot-----------------------------------Positive Control/Check for free movement.

- Objective----------------------------Cancel and “Return to Home” or conduct Forced Landing Procedure.

NOTE

- The flight team will then investigate the reasons why the failure occurred, log the incident, and report it to the UAS Accountable Manager.

## 6.7 Aircraft Battery Failure

The following is the procedure to be followed when an aircraft battery failure has been detected.

The UAS Operator will conduct the following actions.

DETECTION

- UAS Telemetry show decrease and increase of Battery Voltage.

- UAS May decrease in Altitude and Hold Position.

- UAS Telemetry abnormal power surges in ESC’s.

SIGNIFICANCE

- Manual control over UAS by Pilot.

- Monitor telemetry for Battery Spiking.

IMMEDIATE ACTIONS

| Remote Pilot | Actions |
| --- | --- |
| - Remote Pilot-------------------- - Altitude--------------------------- - Flight Mode--------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- | 1. Call “Battery Failure” 2. Check to maintain present altitude 3. Check for GPS Mode is selected 4. Take Positive Control of UAS 5. Drop Flight Height to Safe Height 6. Return to Home Position Manually |

NOTE

- The flight team will then investigate the reasons why the failure occurred, log the incident, and report it to the UAS Accountable Manager.

## 6.8 Loss of GPS Signal

The following is the procedure to be followed when the GPS Signal has been lost.

The UAS Operator will conduct the following actions:

DETECTION

- UAS Drifts with Wind.

- UAS does not Hold Position.

- UAS Telemetry shows No GPS Connection.

SIGNIFICANCE

- Manual control over UAS by Pilot.

- Monitor telemetry for GPS reconnection.

IMMEDIATE ACTIONS

| Remote Pilot | Actions |
| --- | --- |
| - Remote Pilot-------------------- - Altitude--------------------------- - Flight Mode--------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- | 1. Call “GPS” Failure 2. Check to maintain present altitude 3. Check to see if ATTI Mode is selected 4. Take Positive Control of UAS 5. Drop Flight Height to Safe Height 6. Return to Home Position Manually |

ACTIONS

- GPS connection-------------------Confirm Connection Established.

- UAS-----------------------------------Positive Control.

- Pilot-----------------------------------Positive Control/Check for free movement.

- Objective-----------------------------STOP mission flight

- Pilot-----------------------------------Proceed with “Return to Home”

NOTE

- The flight team will then investigate the reasons why the failure occurred, log the incident, and advise the Accountable Manger.

## 6.9 Low Magnetic Field

The following is the procedure to be followed when a Magnetic Field has been detected.

The UAS Operator will conduct the following actions.

DETECTION

- UAS will move from its GPS Hold.

- UAS responses will be random, erratic & Fast.

- UAS Telemetry shows Magnetic Field.

SIGNIFICANCE

- Manual control over UAS by Pilot.

- Monitor telemetry for further Information.

IMMEDIATE ACTIONS

| Remote Pilot | Actions |
| --- | --- |
| - Remote Pilot-------------------- - Altitude--------------------------- - Flight Mode--------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- | 1. Call “Magnetic Field” Failure 2. Check to maintain present altitude 3. Check for GPS Mode is selected 4. Take Positive Control of UAS 5. Increase Flight Height to Safe Height to clear Magnetic Field. 6. Return to Home Position Manually 7. Re-Calibrate Compass Free from Magnetic/Metal objects |

ACTIONS

- UAS-------------------------------- Test Flight to be conducted.

NOTE

- The flight team will then investigate the reasons why the failure occurred, log the incident.

## 6.10 Public Encroachment

The following is the procedure to be followed when there has been encroachment by the public.

The UAS Operator will conduct the following actions:

DETECTION

- People directly under UAS.

SIGNIFICANCE

- Manual control over UAS by Pilot.

- Monitor telemetry Information.

IMMEDIATE ACTIONS

| Remote Pilot | Actions |
| --- | --- |
| - Remote Pilot-------------------- - Altitude--------------------------- - Flight Mode--------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- | 1. Call “public in Area of Operations” Failure 2. Check to maintain present altitude 3. Check to see if ATTI Mode is selected 4. Take Positive Control of UAS 5. Drop Flight Height to Safe Height 6. Return to Home Position Manually |

ACTIONS

- UAS----------------------------------Observer to approach People stating the Dangers of the UAS in flight.

- Pilot----------------------------------Once the area is cleared, return to Mission.

- Observer----------------------------Monitor the Environment

NOTE

- The flight team will ensure prior to the continuation of the task the area is re-assessed and more preventative control measures are put in place.

## 6.11 Aircraft Incursion

The following is the procedure to be followed when there has been an aircraft incursion or suspected incursion. This will take place if another aircraft has been seen below 400ft (120m) within your operating area. The UAS Operator will conduct the following.

DETECTION

- Distance Sound of Manned Aircraft approaching.

- Manned Aircraft “Appears” in front of you.

- Manned aircraft identified

SIGNIFICANCE

- Manual control over UAS by Pilot.

- Avoid Manned Aircraft.

IMMEDIATE ACTIONS

| Remote Pilot | Actions |
| --- | --- |
| - Remote Pilot-------------------- - Altitude--------------------------- - Flight Mode--------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- - Remote Pilot-------------------- | 1. Call “Aircraft Seen North/South/East/West” 2. Drop present altitude to a lower safer Alt. 3. Check in GPS Mode if not, select it. 4. Take Positive Control of UAS 5. Drop Flight Height to Safe Height 6. Return to Home Position Manually |

ACTIONS

- UAS--------------------------------------Positive Control.

NOTE

- Once a flight is complete both the UAS Operator and Observer are to fill out the ‘Incident Form’ and pass it onto the Accountable Manager.

- Accountable Manager will then investigate further and report to IAA.

## 6.12 Fly Away

The following is the procedure to be followed when the UAS is not adhering to the course plotted or as controlled by the UAS OPERATOR. The UAS Operator must conduct the following actions.

DETECTION

- UAS Continues on a course not controllable by the Pilot.

- UAS does not respond to any commands

- UAS Telemetry shows UAS continuing on heading.

SIGNIFICANCE

- Manual control over RPAS/UAS by Pilot.

- Monitor telemetry for further Information.

IMMEDIATE ACTIONS

| Remote Pilot | Actions |
| --- | --- |
| - Remote Pilot-------------------- - Altitude--------------------------- - Flight Mode--------------------- - Flight Direction----------------- - Battery Voltage----------------- - Remote Pilot-------------------- - Remote Pilot-------------------- | 1. Call “Fly Away” 2. Present altitude record. 3. Check in GPS Mode if not select it. 4. Record Direction of flight. 5. Record Battery Voltage 6. Activate Return to Home function. 7. Visually monitor Drone flight direction |

ACTIONS

| Remote Pilot | Actions |
| --- | --- |
| - Remote Pilot/Crew------------ - Altitude--------------------------- - Flight Mode--------------------- - Flight Direction----------------- - Battery Voltage----------------- - Remote Pilot/crew------------- - Remote Pilot/crew------------- | 1. Date Time Group (e.g 071030122021) 2. Present altitude record. 3. Estimated Battery Life/Flight time remaining 4. Record Direction of flight. 5. Record Battery Voltage 6. Contact ATC. 7. Assist the UAS Operator to find system |

NOTE

- Both parties must complete the ‘Incident Form’ within 12hours of the failure and it must be submitted to the UAS Accountable Manager.

- If the system is found both parties must conduct the post-crash management procedure. If the system is not found the UAS Accountable Manager must be notified asap

## 6.13 Pilot Incapacitation

The following is the procedure if the UAS Operator at any time feels unwell or incapable of completing his task. He must conduct the following.

DETECTION

- UAS maintains heading.

- UAS Pilot States he is not feeling well.

- UAS Observer Gets Ready for a handover.

SIGNIFICANCE

- If Observer does not take over control of the UAS. It will fall out of the air after battery is depleted.

- Monitor telemetry for further Information.

IMMEDIATE ACTIONS

| Remote Pilot | Actions |
| --- | --- |
| - Remote Pilot-------------------- - Remote Pilot ------------------- - Remote Pilot ------------------- - Crew/Remote Pilot------------ - Crew/Remote Pilot------------ - Remote Pilot-------------------- | 1. Must State that they are not feeling well, if time permits 2. If possible, Trigger Return to home function. 3. If Possible, Hand over to Crew Member. 4. Visual on Drone, Monitor RTH action. 5. Remote Pilot to Sit down close to Crew member. 6. after RTH completed Crew member to Administer First Aid if need be or call emergency services. |

ACTIONS

UAS--------------------------------------------Positive Control

NOTE

- If medical attention is required, the Observer will call the local hospital and manage the area.

- If during the flight, there is a flight safety risk the Observer will take control of the system and land it.

- During the observer course this action will be tested. The UAS Accountable Manager is to be informed immediately.

## 6.14 Fire Ground Equipment

The following is the procedure will be implemented if there is a fire in any of the ground equipment. The UAS Operator will conduct the following actions.

DETECTION

- UAS Ground Station will have smoke or fire coming from it.

- UAS if losses Communications it will Return to Home.

- UAS Telemetry shows Return to Home Function activated.

SIGNIFICANCE

- Ground Station Unusable.

- Monitor telemetry for further Information.

IMMEDIATE ACTIONS

| Remote Pilot | Actions |
| --- | --- |
| - Remote Pilot-------------------- - Ground Crew------------------- - Ground Crew------------------- | 1. Call “Fire, Fire, Fire” 2. Ready Fire Fighting Equipment. 3. If Fire gets out of control Call Emergency Services. |

ACTIONS

- UAS--------------------------------------Monitor while UAS is Landing.

The Observer will conduct the following actions.

- Observe the fire, ensuring it is not getting out of control

- Secure the area

- Manage any other parties in the area.

NOTE

- An incident form and equipment form must be filled out and sent to the UAS Accountable Manager as soon as possible.

## 6.15 Fire In Flight

The following is the procedure will be implemented if there is a fire in flight. The UAS Operator will conduct the following actions.

DETECTION

- UAS smoke or flames coming from aircraft.

- UAS does not respond to commands.

SIGNIFICANCE

- Manual control over RPAS/UAS by Pilot.

- Monitor telemetry for further Information.

IMMEDIATE ACTIONS

| Remote Pilot | Actions |
| --- | --- |
| - Remote Pilot-------------------- - Flight Mode--------------------- - Ground Crew------------------- - Ground Crew------------------- - Ground Crew------------------- | 1. Call “Fire, Fire, Fire” 2. Check and Immediately Go to Forced landing Procedure. 3. GPS mode, Check - LAND ASAP. 4. Ready Fire Fighting Equipment 5. If Fire gets out of control Call Emergency Services. |

ACTIONS

- Once the aircraft is on the ground the flight team will monitor the fire and start the incident form.

- The UAS Accountable Manager must be informed asap.

- If the fire starts to raise concern, then the emergency services are to be notified.

# 7. Emergency response plan (ERP)

This is a procedure which is applicable to most emergency situations

Remote Pilot

1. All flight operations at the operating area shall Stop. 2. Identify/Locate the incident site. 3. Establish communication to local ATC if required to inform them of the emergency and its status including all relevant telemetry, geographical and technical information as required. This is only if ATC need to be Notified as in if you are operating within Controlled Airspace or within the vicinity of Controlled Airspace. 4. Assess the situation prior to deciding if ATC need to be notified. 5. Contact relevant emergency services if necessary. 6. Secure the area surrounding the incident and ask/instruct any people unrelated to the operations to leave the area. 7. If staff are a first aid responder administer first aid to injured personal. 8. Ensure the public is kept clear of the incident site. 9. Identify any dangers or risks at the incident site and isolate and make safe if appropriate to do so. 10. Check on Injured parties. If further first aid is required apply. 11. Remote Pilot to keep communications with the affected stakeholders open. They include but are not limited to. a. ATC b. Police c. Fire Brigade d. Coast Guard e. Ambulance f. NAA g. Landowners Any other relevant site officials

## 7.1 UAS Loss of Containment Emergency Actions

In the event of an emergency, safety is the number one priority and preventing escalation.

Remote Pilot

1. Follow the “Fly away” procedure outlined in the Emergency Procedures. 2. Notify ATC and the relevant authorities/emergency services and follow all instructions given by them. 3. Liaise with other emergency services as necessary. 4. Inform Your Company Name Safety Manager and Accountable Manager of the Emergency. (They maybe the last person you notify, Emergency Services First.)

## 7.2 Immediately Post-Incident Actions

When the Emergency Response Plan is being triggered the following should be carried out once all danger to life or property has been negated.

Remote Pilot

1. Do Not Move anything. Slowly move into incident Site. Take a mental picture for later recall. If you have time quick sketch of area will help for reference later. 2. Record the incident in detail, record weather conditions as this will be vital for investigation later. 3. Document the situation with photos and videos. You can never take enough Photographs. One or two Photographs is not enough. You will need to cover the Incident area in detail with photographs. Then move to the aircraft crash site. Pay attention to damage to the UAS, damage to property/personnel, record locations and pieces of the UAS. 4. Gather contact information if the damages involve people or private property. If incident involves the likes of a pole, tree, ground, Wall, Building front, etc. Record and take photos of the site where the drone impacted. Even if no damage is visible. Record the Date & Time of when Photographs were taken and draw rough sketch. 5. Complete an incident / accident report. If possible, extract all necessary Logs and technical information from the UAS equipment. 6. Remove UAS from service if it is unsafe to fly and requires maintenance. 7. File all the necessary reports as per the Occurrence Reporting.

## 7.3 Environmental Emergency Actions

In the case of all environmental emergencies the following steps should be taken.

Remote Pilot

1. If possible, try to contain emission or spill immediately to avoid any further environmental harm. Remember Scene Safety and your Safety is First above anything else. If you do not have the correct PPE or equipment to deflect any Spill do not attempt. 2. If not trained to use shut-off vales don’t attempt to do so call site safety personal. 3. Raise the alarm locally to ensure further containment measures can be put in place or emergency response if required. 4. Your Company Name must report all significant incidents immediately to the local authorities and EPA to ensure regulatory compliance and initiate the appropriate response from those agencies. 5. Record with the use of Photographs and Sketch, record Wind direction and Speed. 6. Remember date time is also needed to be recorded write a brief outline of what happened. Bullet points are better in these circumstances than a detailed report. 7. Once back in office or at home a detailed report must be completed as the incident is still fresh in your memory. Do not put this report off until the next day.

## 7.4 UAS Loss of Containment Emergency Actions.

In the event of an emergency safety is the number one priority and preventing escalation.

Remote Pilot

1. Follow the “Fly Away” procedure outlined in the Emergency Procedures. 2. Notify ATC and the relevant authorities/emergency services and follow all instructions given by them. 3. Liaise with other emergency services as necessary. 4. Inform Your Company Name Safety Manager and Accountable Manager of the accident. 5. Once positive control is lost from the aircraft record its direction, speed, Battery voltage remaining, with DJI Products you will have actual approx. time remaining record this. Flight Height, Wind speed, Wind Direction, & Drone Speed.

# 8. Security

The relevant security aspects, including areas of application, safety measurements, procedures and responsibilities is described below:

## 8.1 Operational

In order to ensure that security requirements applicable to the area of operations are complied with in the intended operation, the following measures are taken:

- The take-off location is as secure as possible without infractions to public safety or population.

- The take-off location is monitored by the crew members, and in particular by the Remote Pilot.

- In case that the attendance of people at the take-off site or it is to difficult to manage by the Remote Pilot, the assistance of a VO shall be requested.

- All personnel involved in the operation, including visitors must wear a Your Company Name high visibility jacket.

- Remote Pilot must be easily identifiable.

In order to protect the operation against unlawful interference and unauthorised access, the following measurements are to be taken:

- Company Phones to have a secure lock to avoid Security Breach of data.

- Company laptops are protected with password to avoid Security Breach of data.

- Both the primary as well as the secondary telemetry links are protected with encryption from the Manufacturers. (DJI & Sensefly have encryption end to end technology during Flights.)

- Both the UAS and RP are under the constant watch of one of the crew members.

- The internet wireless Hotspot is protected with password.

- Company Wi-fi Dongle has a password to prevent to avoid Security Breach of data.

If the Remote Pilot is integrated inside a van, This is to isolate the crew from external distractions. This is not Normal Practice and should only be use if no other option is available. If there is a breach of Data, this must be reported to Your Company Name GDPR appointed compliance officer.

## 8.2 Data Protection

The UAS operator is responsible for complying with any applicable European Union and national rules, in particular, regarding privacy, data protection, liability, insurance, security and environmental protection. All operations will be designed to ensure that the UAS operations are in compliance with Regulation (EU) 2016/679 on the protection of natural persons regarding the processing of personal data and on the free movement of such data.

Personal data — Personal data is any information that relates to an individual who can be directly or indirectly identified. This includes

- Photograph

- Names

- Email addresses

- Location information

- Ethnicity, gender, biometric data, religious beliefs, web cookies, and political opinions.

Personal Data which is captured, and we have be given the permission to use this personal data, can only be used for which the permission was given. Once captured with permission still is personal data, permissions do not remove a persons rights to personal data or their right to privacy.

Personal Data has been captured by a UAS operation if the data displays any of the following:

- A person’s clearly visible face. However, if there are individuals in the distance and the faces are blurred, it is unlikely to be considered personal data.

- Any other personally identifiable attributes in another way – from the location, visible address numbers, car plate numbers, time of day, specific clothing, etc.

- It shows details about a person’s bodily characteristics, behaviour, private life or his or her professional activities.

- It is used when making decisions on how to treat, act towards or evaluate an individual person.

- It focuses on or targets a particular person, especially if for a prolonged period of time.

Data processing — Any action performed on data, whether automated or manual. examples include:

- Collecting

- Recording

- Organizing

- Structuring

- Storing

- Erasing

Data subject — The person whose data is processed. These include:

- Customers/Clients

- Site visitors

#### 8.2.1 Definitions

Data controller — The person who decides why and how personal data will be processed. This will be carried out by a person nominated by Your Company Name.

Data processor — A third party that processes personal data on behalf of a data controller. For Data controllers a GDPR compliance checklist is available here https://gdpr.eu/checklist/

# 9. Guidelines to minimise nuisance and environmental impact referred to in UAS.SPEC.050(a)(v);

In order to minimise nuisances, including noise and other emissions-related nuisances, to people and animals, the RC and the crew shall:

- Wear corporate clothing to be easily identified and related with the operation by any external party.

- Sensitise communities about the nature and goals of the project, giving introductions to the UAS and operations and even exercising demonstration flights.

- Explain about the nature of the operations and UAS to any external party that may show interest or request information. The information shared must only be of general knowledge.

- If any external third party complains about the operation and wants to see the operations permissions, you are welcome to do so.

- Address to external parties in a calm and polite way. If the situation gets complicated call the police, retire and stay calm.

- Do not execute manual flights against or above unrelated people or publicum.

If any client, or manager sets you under pressure and you are uncomfortable with the operation do not face them directly. Refer them to the FOM instead and avoid direct conflict.

# 10. Occurrence reporting procedures according to Regulation (EU) No 376/2014.

Drones, are subject to accident investigations and occurrence reporting schemes. In the same manner as Manned Aircrafts. Voluntary or Mandatory reporting should be carried out using the reporting processes provided by the competent authorities (ECCAIRS). According to Regulation (EU) No 376/2014 as a minimum, this should contain:

Reporting procedures in case of:

- damage to property.

- a collision with another aircraft.

- a serious or fatal injury (third parties and own personnel).

The purpose of occurrence reporting is to improve the safety and reliability of aircraft and their operation and thereby to avoid accidents and serious incidents.

## 10.1 Reporting

In case of occurrence the RC must send a detailed occurrence report to the safety manager, the following documentation:

- Detailed description of the occurrence.

- Flight log (if the occurrence took place during flight).

- Telemetry log (if the occurrence took place during flight).

- Explanatory pictures.

  - Touch nothing

  - Do not move any parts or pieces

  - Take more than 20 images using your phone or Camera from different angles.

  - One or two Photos will not be enough.

The purpose of these internal reports is to identify safety related occurrences in operations and classify them accordingly for development of risk mitigation strategies and analysis.

#### 10.1.2 Definitions

- Accidents, according to definition of 1.1.1 Definition of Terms

- Incidents, according to definition of 1.1.1 Definition of Terms

## 10.2 Starting a report.

Obligations to report apply in accordance with Regulation (EU) No 376/2014, see article 3(2). If the incident/accident/crash resulted in a fatal or serious injury to a person, or it involved manned aircraft then it is to be considered to be a serious incident/accident and must be reported. Any occurrence relating to the UAS operation that resulted in significant property damage and/or injury or the potential of injury to crewmembers, clients or members of the public should be reported.

Any occurrence resulting in a collision or near miss with another aircraft must be reported.

  - Weather

  - Mission planning

  - Pilot errors

  - Wrong procedures

  - Defect of UAS

  - Abnormal or erratic behaviour of the UAS

  - Hardware malfunctions, damages, or defects before starting operations

  - Software malfunctions or abnormal messages or behaviours

#### 10.2.1 Requirements

Your Company Name has regular reporting requirements and accident incident-based reporting requirements. Incident and accident reports should flow from the Remote Pilot through the Safety Manager, but both have time limits on their submission. In the event it is not possible to coordinate with the Safety Manager, the Remote Pilot must submit them directly and follow-up with the Safety Manager as soon as possible.

## 10.3 Reporting Procedure.

Remote Pilot

1. Complete an incident/accident report (Appendix 3) onsite immediately post-accident/incident. 2. Notify the Safety Manager immediately with this information: a. Name of the RC. b. Brief summary of the accident, including if any person or property was damaged. 3. In the case of an Accident or Incident: Within 12-hours send the completed Incident / Accident Report (Appendix 3) to the Safety Manager. 4. In case of Event of Interest: The Safety Manager will determine whether Accident Incident/Event of Interest Report Form needs to be completed and/or any other reports.

Safety Manager Actions

1. Submit an accident report with the IAA within 10 calendar days from the time of the accident using the European Aviation Reporting Portal (https://aviationreporting.eu/) If operating in another EU State this portal is to be used also, but RP must report to NAA of incident. 2. Accidents/serious Incidents involving the operation of a UAS must also be reported to the Air Accident Investigation Unit of the Department of Transport (info@aaiu.ie) and to the Flight Operations Department of the IAA in accordance with regulation or guidance material issued by the IAA (Mandatory Occurrence Reporting protocols). 3. Following an accident, incident or event of interest, the UA Operations Manager and relevant parties will debrief the RC and any crew members, 4. Confirm the following was in place at the time of the accident, incident or event of interest. a. RC held the appropriate certification. b. Approval was granted by the UA Operations Manager for the mission location, date and time. c. RC was complying with the conditions of the permissions UAS operations were under. d. UAS that was involved was approved by the UA Operations Manager to be used. 5. UAS that was involved was operated in compliance with this OM, IAA regulations and with permission of local ATC (as applicable). 6. UAS that was involved was insured.

Your Company Name shall cooperate fully with the accident investigation body, competent authority, and other government entities (e.g., Gardai, Dept Of Transport, IAA) in relation to post accident investigations. All information, Personal Statements, Witness Statements pertaining to the accident/incident if requested will be shared with the relevant authorities in hard copy or soft copy. Any surviving flight logs or technical information from the UAS.

# 11. Record-keeping procedures

Description of instructions on logs and records of pilots and other data considered useful for the tracking and monitoring of the activity. The Following Logs are just suggestion only. If you have your own, you can insert a copy of them here. You must have at a minimum Pilot Logbook and Drone Flight Logbook.

## 11.1 Pilot Logbook

| Pilot Logbook |  |  | Jan-2021 to Dec-2021 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | UAV Type | Uav Ser Number | Location | Total Flight Time | Manual | Auto | Training- Test Flight | Client base Flight Job No |  |
| 0721 | PH4 | KL0871 | Dublin | 00:25 | YES | NO | Training |  |  |

Table 12: Pilot Logbook

Summary: Last Page Hours Carried over __________________________

Total Flight Hours this page: ____________________________________

Signature___________________________________________________

## 11.2 Flight Logbook

| Place Company Logo Here |  |  | Contact Details Accountable Manager: Chief Remote Pilot: Remote Pilot: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Payload | Operator 1 | Operator 2 | Site manager | Location | Launches | Time of Flight | Environment Wind/Temp |  |
| Serial Number | Record of Defects |  | Serial Number | Maintenance Carried out |  |  | Worked Done By | Date | Site Manager |
| Signature Block to be signed when all flight mission is filled. |  |  |  |  |  |  |  |  |  |
| Operator Signature: |  |  |  |  |  |  |  |  |  |
| Any Flight Issues: |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  |  |  |  |  |

Table 13:: Flight Logbook

## 11.3 Battery Logbook

| Battery Logbook. |  |  |  |  |  |  | Page Number: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Battery Ser No. | Start Charge Time | Start Voltage | Finished Voltage | End Charge Time | Total Charge Time | Signature | Remarks |
| Example | 100201 | 1630hrs | 25.3v | 29.5 | 0830hrs | 16 hours | Wayne Floyd | NTR |

Table 14:Battery Logbook

## 11.4 Drone Maintenance Logbook

| Maintenance Logbook |  |  |  |  | Page Number: |  |
| --- | --- | --- | --- | --- | --- | --- |
| Date | Reason For Maintenance | Work Completed | Completed By | Parts Replaced | Test Flight Signature | Notes |

| Aircraft: (Serial No if required) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Model: |  |  |  |  |  |  |
| Firmware Version: |  |  |  |  |  |  |
| Software Version: |  |  |  |  |  |  |
| Date | Rotors | Motors | Controls Calibration | IMU calibration | Structural Check | Inspected by |

Table 15: Maintenance Logbook

Last Page of Operations Manual

Insert Company Logo here